|<

Control System

CMC <> Camera AND VMS System Interface Specification RS422

SorterCMC.P10a

Journal:	p10a_OH_Scanner&VMS_RS422.docx

Date:	03 Jan 2024

Revision:	1.5

Pages:	20

Initials:	PGP

© BEUMER Group 2021 This document and all data contained herein are the exclusive intellectual property rights of BEUMER Group and are supplied in strictest confidence under the express condition that any reproduction, transmission, transcription etc., or storing in any retrieval system/form, or any kind of supply to others is strictly forbidden without express prior written consent. The receiver guarantees that any disclosed material shall not be used in any way detrimental to the interest of BEUMER Group.

|<

Contents

Page

1.	revisions	4

2.	Introduction	5

2.1	About this Document	5

2.1.1	Target group	5

2.2	​​​​​​​Abbreviations and Acronyms	5

3.	SURVEY OF SYSTEM	6

3.1	SYSTEM CONFIGURATION	6

3.2	HARDWARE DESIGN	6

3.2.1	Speed signal and read signal	6

3.2.2	Communication	7

4.	TIMING	8

4.1	Speed Signal	8

4.1.1	Speed signal accuracy	8

4.2	Read Signal	9

4.3	TELEGRAMS	9

5.	INTERFACE	10

5.1	IN GENERAL	10

5.1.1	Error messages	10

5.1.2	Telegram format	10

5.1.3	Filler	11

5.1.4	Separator	11

5.2	COMMUNICATION PROTOCOL	11

5.2.1	Initializing telegram to scanner CMC Scanner	11

5.2.2	Initializing telegram to CMC Scanner  CMC	11

5.2.3	Index telegram CMC Scanner	12

5.2.4	Code telegram Scanner  CMC	12

5.2.5	Error telegram	13

5.2.6	Heartbeat Telegram	14

6.	DEVIATIONS FROM NORMAL OPERATION	15

6.1	Scanner Does Not Reply to Initialization	15

6.2	Code Telegram with Index 9999	15

ENCLOSURE 1: ASCII CODES	16

ENCLOSURE 2: Error Types for Scanner	18

# revisions

| Rev. | Date | Init. | Description |
| --- | --- | --- | --- |
| 1.0 | 31 Jan 2018 | CHD | Document created from 12324 Nacex Rev. 1.0 Removed project specific comments |
| 1.1 | 06 Jun 2018 | ADJ | Updated header/ footer. Updated 5.2.5, 5.2.6 reference enclosures |
| 1.2 1.3 | 05 Oct 2021 04 Aug 2023 | PGP DTP | Enclosure 2 updated with 31-37 Hex Alarms Includes VMS for DHL E-Com projects |
| 1.4 | 25 Aug 2023 | DTP | Update Error types with item overweight |
| 1.5 | 03 Jan 2024 | DTP | Update Error types with extra errors from Sick, removed overweight item error (38) -signal is sent through hardwire from scale to induction |

# Introduction

The purpose of this description is to document the communication process and the telegram formats between the Crisplant Machine Controller (CMC) and the camera and volume measurement system.

The camera and volume measurement system is connected to both the CMC via serial communication line (RS422) and the CSC via Ethernet.

The CMC sends an index telegram when an item is to be scanned and measured. The camera and volume measurement system answers the CMC with a code telegram, which does not contain any barcode and dimension information. The read signal is propagated to the camera and dimensioning unit internally by camera controller unit. The camera and volume measurement system communicates directly with the CSC with regards to the barcode and item dimension information.

Please note: In the following sections, camera and volume measurement system is referred to as scanner. The CMC and the scanner – camera and volume measurement system have only one interface. The scanner controller will distribute and gather all information  to / from camera and VMS internally. Interface descriptions and examples in sections below are between CMC <> scanner.

## About this Document

### Target group

CMC and commissioning engineers and scanner suppliers

## ​​​​​​​Abbreviations and Acronyms

CMC	Crisplant Machine Controller

CSC	Crisplant System Controller

MAI	Manual Induction Unit

PEC	Photo Electric Cell

RNI	Ready Next Item

SPS	Stray Parcel Supervision

VMS	Volume Measurement System

# SURVEY OF SYSTEM

## SYSTEM CONFIGURATION

The CMC to scanner communication is composed of three parts: a 24 VDC speed signal, a 24 VDC scanner read signal, and a serial communication line.

Figure 3-1

## HARDWARE DESIGN

### Speed signal and read signal

The electrical source of the speed signal and the scanner read signal is a digital output in the CMC. The output of this unit has the following specifications:

Logic on – "activated":	24VDC

Logic off – "deactivated":	0VDC

Max. allowable output current:	130mA

Fig. 3-2

### Communication

The CMC communicates with the scanner by means of two RS422 point-to-point asynchronous serial communication lines, thus achieving a 4-wire full duplex communication line. Transmitters are always enabled.

Information frame

A character has to be a 7-bit ASCII code:

1 start bit

7 data bits

1 parity bit - even parity

1 stop bit

Transmission speed

Transmitting and receiving must take place at the same transmission speed:

9600 Baud

See also the RS485 Standard.

# TIMING

## Speed Signal

The CMC generates the speed signal for which the frequency is proportionate to the velocity at which the carts run.

A speed signal period corresponds to a cart movement of 50 mm. The tolerance for a speed signal period is ± 15 mm at a speed of 1.5 m/Sec.

A complete speed signal period is defined as a "tach".

The minimum high pulse width is 5 msec.

──┐  ┌──┐  ┌──┐  ┌──┐  ┌──┐  ┌──┐  ┌──┐  ┌──┐  ┌──┐  ┌──┐  ┌──

└──┘  └──┘  └──┘  └──┘  └──┘  └──┘  └──┘  └──┘  └──┘  └──┘

│<   >│

A tach: 50 mm  (2.5 mm + Xmax1) [sec] * sorter speed[mm/sec])

*1) see section 4.1.1.1 for Profibus

For the LS-4000 generation sorters, the tach will always be 50mm.

### Speed signal accuracy

#### Profibus

The signal is generated based on the clock pulses which are read from the IO image every two msec. The IO image is updated by the Profibus controller for every Profibus scan. The Profibus scan time is not cyclic but rather changing depending on the load of the Profibus (DPV0 and DPV1 telegrams). The minimum cycle time will normally be (depending of no. of nodes and size of the IO) between 2 and 7msec. So due to the nature of the system, a jitter of the speed signal must be expected. As a minimum, the jitter will fluctuate between 4 and 16 msec (scan time + 2*Profibus scan time) – but is not limited to this value; in some instances, the value will be higher such as when nodes are switched off or at high loads of DPV1 communication. In both cases, the system is not operating in normal mode and is likely either stopping due to errors or not running at all.

If the jitter is unacceptable from the scanner system’s point of view, steps must be taken to obtain a better accuracy through an external system.

Xmax = 16 msec.

## Read Signal

CMC:

The timing of the read signal from the CMC to the scanner unit can be seen from the below figure:

Fig. 4-1

The figure illustrates scanning of one- and two-tray items:

- The distance D1 is the delay from the front of the tray to the scanner read signal is activated. The distance D1 is min. 0 and is adjustable in steps of whole tachs.

- The distance D2 is the distance from the scanner read signal is deactivated to the end of the tray. The distance D2 is min. 0 and is adjustable in steps of whole tachs.

- The read signal deactivated distance D3 will be at least 1 tach.

- The read signal activated distance D4 will be at least 1 tach. D4 has to be maximized in relation to D3.

The distances mentioned above are all measured as counts of tachs (50 mm). Therefore, if the distances are converted to time, these will depend on the velocity of the sorter.

At a sorter velocity of 1.5 m/sec, one tach corresponds to 35 msec.

## TELEGRAMS

The following describes the normal communication process after the scanner has been initialized.

The CMC will transmit the index telegram to the scanner 2 tachs after the rising edge of the read signal or at the middle of the read signal activated period whichever is the smallest.

The transmission of the index telegram will be completed before the read signal is deactivated.

# INTERFACE

The CMC is the master of communication between the CMC and the scanner. Thus, the CMC commands the scanner to read.

Below is an example of correct communication:

Communication	Description

CMC— S  Scanner	The CMC transmits an index number after the read signal has been activated.

CMC C — Scanner	The scanner answers the CMC with a code telegram.

## IN GENERAL

### Error messages

Even though the CMC is the master of communication, the scanner can transmit error messages. These messages are transmitted spontaneously when an error is detected in the scanner.

### Telegram format

The format of the telegrams is shown below. Whether the data field is used depends on the telegram type:

Telegram format	<STX><TYP><DATA><ETX><CHKSUM><CR><LF>

<STX>	Frame beginning. The ASCII character 02 HEX.

<TYP>	Telegram type. One ASCII character with a value between 07 HEX and 7F HEX.

<DATA>	Data field. ASCII characters with values between 07 HEX and 7F HEX.

The maximum length of the data field is 200 characters.

An even number of characters is always transmitted.

<ETX>	Frame end. The ASCII character 03 HEX.

<CHKSUM>	The checksum is 7F HEX less the rest of the following:

-The sum of all characters in the telegram exclusive of STX, ETX and CHKSUM Modulo 7F HEX (00 HEX ... 7E HEX).

(Modulo is similar to the rest of an integer division).

<CR>	0D HEX

<LF>	0A HEX

All telegrams are terminated with <CR><LF>. In the following examples, the termination characters are not shown.
Example of an initializing telegram with checksum:

Telegram type:	I

Telegram format: 	<STX><I><ETX><CHKSUM>

| Bit 15 | Bit 0 |  |
| --- | --- | --- |
| 02 HEX | 49 HEX | DW0 |
| 03 HEX | 36 HEX | DW1 |

### Filler

The introduction of a filler is necessary in order to ensure that the codes are always located in whole 16-bit data words.

The filler is:		# (23 HEX)

### Separator

In order to separate the data field, which contains information about the code length, from the part that contains the codes, a separator is introduced. A filler is always placed after the separator.

The separator is:	, (comma)(2C HEX).

The separator facilitates separation of the code length and the codes.

## COMMUNICATION PROTOCOL

This section describes all telegrams transmitted between the CMC and the scanner.

### Initializing telegram to scanner	CMC Scanner

After a reset and when an error has been detected on the communication line, the scanner receives an initializing telegram. The scanner must clear all internal buffers when this telegram is received. This telegram has the following format:

Telegram type:	I

Telegram format:	<STX><I><ETX><CHKSUM>

| Bit 15 | Bit 0 |  |
| --- | --- | --- |
| STX | I | DW0 |
| ETX | CHKSUM | DW1 |

### Initializing telegram to CMC	Scanner  CMC

In reply to the initializing telegram from the CMC, the scanner unit must transmit an initializing telegram to the CMC. This telegram has the following format:

Telegram type:	J

Telegram format:	<STX><J><ETX><CHKSUM>

| Bit 15 | Bit 0 |  |
| --- | --- | --- |
| STX | J | DW0 |
| ETX | CHKSUM | DW1 |

### Index telegram	CMC Scanner

When an item is to be read, the CMC activates the scanner read signal and transmits the index telegram.

The index telegram has the following format:

Telegram type:	S

Telegram format:	<STX><S><INDEX><GID><ETX><CHKSUM>

| Bit 15 | Bit 0 |  |
| --- | --- | --- |
| STX | S | DW0 |
| L1 | L2 | DW1 |
| L3 | L4 | DW2 |
| G1 | G2 | DW3 |
| G3 | G4 | DW4 |
| G5 | G6 | DW5 |
| G7 | G8 | DW6 |
| G9 | G10 | DW6 |
| ETX | CHKSUM | DW7 |

The GID (Global Item ID) is used by the scanner for unique identification of the item e.g. transmittal of barcodes to CSC or other systems.

The GID is in the range: 0-2.147.483.647

The Index is in the range: 0-1999

### Code telegram	Scanner  CMC

The scanner has to read, handle and send the codes of the barcode(s) on the item(s) being transported on the sortation system.

Please note that the barcode information is sent directly from the scanner system to the CSC through a separate interface; barcode information is not sent to the CMC. The code telegram from the scanner system to the CMC is an acknowledge telegram to inform the CMC that the scanner system has received the index telegram. The code telegram must contain the GID previously received in the Index telegram from CMC.

When the CMC activates the read signal and transmits an index telegram, the scanner unit has to transmit a code telegram.

Note:

The code telegrams are to be transmitted to the CMC in the same order as the index telegrams are transmitted to the scanner.

A principle of the code telegram design is shown below:

Telegram type:	C

Telegram format:	<STX><TYP><INDEX><LEN><SEP>

<GID><ETX>CHKSUM>

<INDEX>	The index number is the number received in the corresponding index telegram.

<LEN>	Length of GID (always 10)

<SEP>	Separator

<GID>	GID received from CMC in index telegram

| Bit 15 | Bit 0 |  |
| --- | --- | --- |
| STX | C | DW0 |
| L1 | L2 | DW1 |
| L3 | L4 | DW2 |
| LEN | LEN | DW3 |
| SEP | FILLER | DW4 |
| G1 | G2 | DW5 |
| G3 | G4 | DW6 |
| G5 | G6 | DW7 |
| G7 | G8 | DW8 |
| G9 | G10 | DW10 |
| ETX | CHKSUM | DW11 |

In case the index telegram has not been received when the read signal is deactivated, the scanner must use the index number 9999 when returning the code telegram. In this case GID is discarded in the code telegram.

### Error telegram

If the scanner detects an error, it is transmitted to the CMC via an error telegram. The telegram design is similar to that of the other telegrams and is shown below:

Telegram type:	F

Telegram format:	<STX><F><ID><ERROR><ETX><CHKSUM>

<ID>	Scanner identity

<ERROR>	Error type

| Bit 15 | Bit 0 |  |
| --- | --- | --- |
| STX | F | DW0 |
| ID | ERROR |  |
| ETX | CHKSUM |  |

The scanner manufacturer, scanner identity, and error types are listed in enclosure 2.

### Heartbeat Telegram

To allow the CMC to regularly check that the connected scanner is operational, the scanner cyclically sends a heartbeat telegram every 10 seconds. If, within a time period of 10 seconds, the scanner has not sent another telegram to the CMC, the scanner will send the heartbeat telegram. If the scanner is defective at the moment when the heartbeat telegram is supposed to be transmitted, the last transmitted error telegram will be resent.

The heartbeat telegram design is similar to that of the error telegram and is shown below:

Telegram type:	F

Telegram format:	<STX><F><ID><DEL><ETX><CHKSUM>

<ID>	Scanner identity

<DEL>	7F HEX

| Bit 15 | Bit 0 |  |
| --- | --- | --- |
| STX | F | DW0 |
| ID | DEL |  |
| ETX | CHKSUM |  |

The scanner manufacturer and scanner identity are listed in enclosure 2.

# DEVIATIONS FROM NORMAL OPERATION

In general, the scanner constantly transmits status information to the CMC, thus ensuring that the CMC knows the status of the scanner.

By means of this information the CMC is able to estimate whether an error is so fatal that it needs immediate response (e.g. stop of the sorter or reset of the scanner).

Telegrams which are considered by the CMC to be invalid are discarded.

## Scanner Does Not Reply to Initialization

Communication	Description

CMC— I  Scanner	CMC transmits initialization to scanner

CMC— I  Scanner	CMC transmits initialization to scanner

CMC— I  Scanner	CMC transmits initialization to scanner

The "I"-telegrams are transmitted at regular time intervals. This interval can be adjusted from 0 to 10 seconds in the CMC.

## Code Telegram with Index 9999

Communication	Description

CMC            Scanner	The scanner does not receive the index telegram as previously described.

CMC  C — Scanner	The scanner transmits a code telegram with index 9999.

Example:

Index number:	99 99

GID:	Discarded

| Bit 15 | Bit 0 |  |
| --- | --- | --- |
| 02 HEX (STX) | 43 HEX (C) | DW0 |
| 39 HEX (9) | 39 HEX (9) | DW1 |
| 39 HEX (9) | 39 HEX (9) | DW2 |
| 30 HEX (0) | 30 HEX (0) | DW4 |
| 2C HEX (,) | 23 HEX (#) | DW5 |
| 03 HEX (ETX) | 26 HEX (CHKSUM) | DW6 |

ENCLOSURE 1: ASCII CODES

| Character | Hexadecimal | Character | Hexadecimal |
| --- | --- | --- | --- |
| NUL | 00 | SP | 20 |
| SOH | 01 | ! | 21 |
| STX | 02 | " | 22 |
| ETX | 03 | # | 23 |
| EOT | 04 | $ | 24 |
| ENQ | 05 | % | 25 |
| ACK | 06 | & | 26 |
| BEL | 07 | / | 27 |
| BS | 08 | ( | 28 |
| HT | 09 | ) | 29 |
| LF | 0A | * | 2A |
| VT | 0B | + | 2B |
| FF | 0C | , | 2C |
| CR | 0D | - | 2D |
| SO | 0E | . | 2E |
| SI | 0F | / | 2F |
| DL | 10 | 0 | 30 |
| DC1 (X-ON) | 11 | 1 | 31 |
| DC2 (TAPE) | 12 | 2 | 32 |
| DC3 (X-OFF) | 13 | 3 | 33 |
| DC4 (TAPE) | 14 | 4 | 34 |
| NAK | 15 | 5 | 35 |
| SYN | 16 | 6 | 36 |
| ETB | 17 | 7 | 37 |
| CAN | 18 | 8 | 38 |
| EM | 19 | 9 | 39 |
| SUB | 1A | : | 3A |
| ESC | 1B | ; | 3B |
| FC | 1C | < | 3C |
| GS | 1D | = | 3D |
| RS | 1E | > | 3E |
| US | 1F | ? | 3F |

| Character | Hexadecimal | Character | Hexadecimal |
| --- | --- | --- | --- |
| @ | 40 | ` | 60 |
| A | 41 | a | 61 |
| B | 42 | b | 62 |
| C | 43 | c | 63 |
| D | 44 | d | 64 |
| E | 45 | e | 65 |
| F | 46 | f | 66 |
| G | 47 | g | 67 |
| H | 48 | h | 68 |
| I | 49 | i | 69 |
| J | 4A | j | 6A |
| K | 4B | k | 6B |
| L | 4C | l | 6C |
| M | 4D | m | 6D |
| N | 4E | n | 6E |
| O | 4F | o | 6F |
| P | 50 | p | 70 |
| Q | 51 | q | 71 |
| R | 52 | r | 72 |
| S | 53 | s | 73 |
| T | 54 | t | 74 |
| U | 55 | u | 75 |
| V | 56 | v | 76 |
| W | 57 | w | 77 |
| X | 58 | x | 78 |
| Y | 59 | y | 79 |
| Z | 5A | z | 7A |
| [ | 5B | { | 7B |
| \ | 5C | ¦ | 7C |
| ] | 5D | } | 7D |
| ^ | 5E | ~ | 7E |
| _ | 5F | DEL (RUB OUT) | 7F |

ENCLOSURE 2: Error Types for Scanner

Error types for scanner identity XX (XX Hex) - XXX

| Error type | Description of error |
| --- | --- |
| 10 HEX | Hardware failure – Top side* / Bottom side** – BX1 |
| 11 HEX | Hardware failure – Left side – BX2 |
| 12 HEX | Hardware failure – Right side – BX3 |
| 13 HEX | Hardware failure – Right side – BX4 |
| 14 HEX | Hardware failure – Left side – BX5 |
| 15 HEX | Hardware failure VMS head – Right side – BX34 |
| 16 HEX | Hardware failure VMS head – Left side – BX35 |
| 17 HEX |  |
| 18 HEX |  |
| 19 HEX |  |
| 1A HEX | Hardware failure – System controller |
| 1B HEX |  |
| 1C HEX |  |
| 1D HEX |  |
| 1E HEX |  |
| 1F HEX |  |
| 20 HEX |  |
| 21 HEX |  |
| 22 HEX |  |
| 23 HEX |  |
| 24 HEX |  |
| 25 HEX |  |
| 26 HEX |  |
| 27 HEX |  |
| 28 HEX |  |
| 29 HEX |  |
| 2A HEX |  |
| 2B HEX |  |
| 2C HEX |  |
| 2D HEX |  |
| 2E HEX |  |
| 2F HEX |  |
| 30 HEX |  |
| 31 HEX | No index telegram received within read signal |
| 32 HEX | Multiple indexes received within read signal |
| 33 HEX | Index already in use |
| 34 HEX | Index telegram error detected (framing/parity or length/ context) |
| 35 HEX | No read signal received for index |
| 36 HEX | Speed signal not present when read signal is active |
| 37 HEX | Telegram with bad checksum received |
| 38 HEX |  |
| 39 HEX | Scale fault reported by scanner communication |
| 3A HEX | No communication between Scanner and Scale |
| 3B HEX |  |
| 3C HEX |  |
| 3D HEX |  |
| 3E HEX |  |
| 3F HEX |  |
| 40 HEX | No scanner in network detected |
| 41 HEX |  |
| 42 HEX |  |
| 43 HEX |  |
| 44 HEX |  |
| 45 HEX |  |
| 46 HEX |  |
| 47 HEX |  |
| 48 HEX |  |
| 49 HEX |  |
| 4A HEX |  |
| 4B HEX |  |
| 4C HEX |  |
| 50 HEX |  |
| 51 HEX |  |
| 52 HEX |  |
| 53 HEX |  |
| 54 HEX |  |
| 55 HEX |  |
| 56 HEX | Trigger failure |
| 57 HEX |  |
| 58 HEX |  |
| 59 HEX |  |
| 5A HEX |  |
| 5B HEX |  |
| 5C HEX |  |
| 5D HEX |  |
| 5E HEX |  |
| 5F HEX |  |
| 60 HEX | No connection to device – Top side / Bottom side – BX1 |
| 61 HEX | No connection to device – Left side – BX2 |
| 62 HEX | No connection to device – Right side – BX3 |
| 63 HEX | No connection to device – Right side – BX4 |
| 64 HEX | No connection to device – Left side – BX5 |
| 65 HEX | No connection to device VMS head – Right side – BX34 |
| 66 HEX | No connection to device VMS head – Left side – BX35 |
| 67 HEX |  |
| 68 HEX |  |
| 69 HEX |  |
| 6A HEX |  |
| 6B HEX |  |
| 6C HEX |  |
| 6D HEX |  |
| 6E HEX |  |
| 6F HEX |  |
| 70 HEX |  |
| 71 HEX |  |
| 72 HEX |  |
| 73 HEX |  |
| 74 HEX |  |
| 75 HEX |  |
| 76 HEX |  |
| 77 HEX |  |
| 78 HEX |  |
| 79 HEX |  |
| 7A HEX |  |
| 7B HEX |  |
| 7C HEX |  |
| 7D HEX |  |
| 7E HEX |  |
| 7F HEX |  |

*Top side for camera tunnels on sorter

**Bottom side for bottom read cameras on inductions

Note: Error types 31Hex to 37Hex are fixed. Moving those errors will have an impact on the CMC.