|<

DHL e-Commerce Dallas and Charlotte hub

CMC Routing

Journal:	p3_Protocol-CMC-Routing DHL eComm DFW & CLT.docx

Date:	10 April 2024

Revision:	1.5

Pages:	107

Initials:	JPK

© BEUMER Group 2020 This document and all data contained herein are the exclusive intellectual property rights of BEUMER Group and are supplied in strictest confidence under the express condition that any reproduction, transmission, transcription etc., or storing in any retrieval system/form, or any kind of supply to others is strictly forbidden without express prior written consent. The receiver guarantees that any disclosed material shall not be used in any way detrimental to the interest of BEUMER Group.

Contents

Page

1.	Revisions	6

2.	Introduction	7

2.1	Optional Telegrams	7

2.2	References	7

2.3	Abbreviations and Acronyms	9

3.	Transport Protocol	10

3.1	Transport Protocols for Siemens S7-1500 PLC Type	10

3.1.1	RFC 1006	10

3.1.2	S7 Functions	11

3.1.3	RFC 793 (TCP Socket)	11

3.1.4	Keep-alive telegram	12

3.2	Transport Protocols for Allen-Bradley Controllogix PLC Type	12

3.2.1	CIP	12

3.2.1.1	Memory layout for reception of telegrams in PLC	12

3.2.1.2	Memory layout for reception of telegrams in CSC/host	13

3.2.1.3	Byte order and exchange of double integers (DINT)	14

3.2.1.4	Framing of telegram blocks in buffer area	14

3.2.1.5	Keep-alive telegram	14

3.3	Hot Standby Enabling	14

3.3.1	Routing receive messages	15

3.3.2	State of standby host/CSC	16

3.3.3	Determining master routing channel	16

3.3.4	Telegram reception in PLC from CSC/Host System 2	16

4.	Application Protocol	17

4.1	Routing Application Telegrams	17

4.1.1	Sequence number (Protocol version 4 only)	18

4.2	Routing	18

4.2.1	Reset request (00)	22

4.2.2	Reset completed (01)	23

4.2.3	Connected (02)	23

4.2.4	Hot standby state (03)	24

4.2.5	Ready (05)	24

4.2.6	Resume (06)	25

4.2.7	Request host standby state (07)	25

4.2.8	Code (10)	26

4.2.9	Set induction unit mode (11)	27

4.2.10	Code acknowledgement (12)	28

4.2.11	Negative acknowledgement (13)	29

4.2.12	Item ID (14)	30

4.2.13	Clear queue (15)	31

4.2.14	Queue cleared (16)	31

4.2.15	Code not used (17)	32

4.2.16	Item ID acknowledgement (18)	33

4.2.17	Code used (19)	34

4.2.18	Induction unit mode status (20)	34

4.2.19	Code – multi dest (21)	36

4.2.20	Item transfer (22) (option)	37

4.2.21	MES code error (23) (option)	39

4.2.22	Code request (24) (option)	40

4.2.23	Code result (25) (option)	41

4.2.24	Scanner result (26)	43

4.2.25	Scale result (27)	44

4.2.26	Volume result (28)	50

4.2.27	Item arrived (29) (option)	55

4.2.28	Chute status (30)	55

4.2.29	Chute status request (31)	56

4.2.30	Block chute (32)	57

4.2.31	Start/stop sorter (35)	58

4.2.32	Start/stop induction unit (36)	59

4.2.33	Start/stop sorter acknowledgement (37)	59

4.2.34	Start/stop induction unit acknowledgement (38)	60

4.2.35	Item inducted (40)	61

4.2.36	Item redirect (41)	62

4.2.37	Item redirect acknowledgement (42)	63

4.2.38	Item discharged (43)	64

4.2.39	Discharge supervision (44)	65

4.2.40	Item sortation event (45)	66

4.2.41	Item disappeared (46) (option)	69

4.2.42	Item redirect-multi dest (47)	70

4.2.43	Carrier passed SPS (48)	72

4.2.44	Carrier data reboot (60)	73

4.2.45	Carrier data reboot acknowledgement (61)	74

4.2.46	Request carrier data reboot (63)	75

4.2.47	Carrier data transmitted (64)	75

4.2.48	Index status (65)	76

4.2.49	Request index status (66)	77

4.2.50	Index data transmitted (67)	77

4.2.51	Input changed (70) (option)	78

4.2.52	Set function (73)	78

4.2.53	Function status (74)	79

4.2.54	Set output (75) (option)	80

4.2.55	Request max. recirculation (90)	81

4.2.56	Max. recirculation (91)	81

4.2.57	Dump chutes (95)	82

4.2.58	Request dump chutes (96)	83

4.2.59	Set status (97)	84

4.2.60	Status (99) (option)	85

4.2.61	Reject telegram (100)	87

5.	Telegram Layout and Scenarios	88

5.1	Telegram Layout	88

5.1.1	Telegram layout – Protocol version 3	88

5.1.2	Telegram layout – Protocol version 4	90

5.2	Telegram Scenarios for Normal Operation	91

5.2.1	Reset/synchronisation scenarios	91

5.2.1.1	CSC/Host start-up – carrier data are downloaded	92

5.2.1.2	CSC/Host start-up – no carrier data are downloaded	93

5.2.1.3	CMC start-up	94

5.2.1.4	CSC hot standby switchover	94

5.2.1.5	CSC hot standby switchover in a reset sequence	95

5.2.2	Standby state	97

5.2.3	Item inducted from induction unit in keyboard mode	97

5.2.4	Disabled induction unit	98

5.2.5	Item inducted from induction unit – OH scanner after code PEC at induction unit	98

5.2.5.1	OH scanner after code PEC at induction unit – induction unit error	99

5.2.5.2	OH scanner at induction unit – no redirection from CSC/Host	99

5.2.6	Item inducted from induction unit in scanner/dispatch mode	99

5.2.7	Item inducted, scanned, sorted and supervised	100

5.2.8	Stray item on induction unit	101

5.2.9	Stray item on sorter inducted	101

5.2.10	Code queue cleared at induction unit	102

5.2.11	Chute full and max. recirculation event for item	102

5.2.12	No activation event for item	103

5.2.13	Disabled carrier	103

5.2.14	Upload of index status	104

5.2.15	Upload of dump chutes status	104

5.2.16	Use of ‘Code-Multi Dest’ telegram	104

5.2.17	Use of ‘Item Redirect-Multi Dest’ telegram	105

5.2.18	Use of ‘Item Transfer' telegram	105

5.3	Telegram Scenarios for Test Operation	106

5.4	Telegram Scenarios for CMC - MES	106

# Revisions

| Rev. | Date | Init. | Description |
| --- | --- | --- | --- |
| 1.13 | 15 Nov 2011 | SFA | Updated document header and dates |
| 1.14 | 30 Aug 2012 | TBS | Changed chute status 64 from "Wedge out fault" to "Possible CTB error" |
| 1.15 | 16 May 2013 | SSJ | Sec 4.2.35 tlg. 40 updated with note for stating that SPS must be in Keyboard mode to induct items with status "Stray". |
| 1.16 | 09 Jan 2014 | HJV | Changed terminology in entire document according to ECN13-004 (new abbreviations and terms). Specifically the term "Induction" has been replaced with the term "Induction Unit". |
| 1.17 | 28 Aug 2014 | PSG | Updated tlg 28 volume result, that now supports both P15 and P17_interface-CMC-VolumeScannerRS422 |
| 1.18 | 02 Jan 2019 | EMO | Updated section 4.2.26 for tlg 28 volume result. |
| 1.19 | 16 Nov 2020 | CHD | Updated section 4.2.60 tlg 99. Added description of possibility for CMC to send tlg 99 for all components during start up. |
| 1.2 | 25 Sept 2023 | DTP | Updated sections 4.2.52 tlg 73 and 4.2.53 tlg 74. Project specific telegrams between CSC and CMC about container type at direct to container chutes |
| 1.3 | January 10, 2024 | JPK | Updating section 4.2.28 Chute Status (30) to include a project specific bit to report missing containers. |
| 1.4 | January 16, 2024 | DTP | Updated section 4.2 – added "DHL" column to the table marking telegrams to be used for DHL project. Reviewed with HLC (Martin Christensen) |
| 1.5 | April 10, 2024 | JPK | Updating section 4.2.28 Chute Status (30) to use bit 4 instead of bit 3 to align with the CMC implementation. |

# Introduction

This document describes the application telegrams used between the CMC and the CSC or an external host.

The application protocol is used to carry command telegrams from the CSC or an external host to the CMC, and the response and information telegrams from the CMC back to the CSC or external host.

This document is divided into two main part. The first part (sections 2 through 4) describes the application telegrams and the second part (section 5) shows different Application Telegram Scenarios.

## Optional Telegrams

Telegrams marked as “option”, are telegrams, which are not supported by the base versions of the CSC and/or the CMC software. That means that they are either not implemented or have to be modified according to the specific project requirements.

## References

[RFC1006]	RFC 1006

Protocol Specification

Platform IO

Journal io.p2

[Fetch/Write]	Simatic Net

NCM for Industrial Ethernet

Manual for NCM S7 V5.1 or higher

Journal: cmc5base3.0.Siemens_FetchWrite_S7wbie2b.pdf

[S7-SAPI]	Simatic Net

S7 Programming Interface

Journal: cmc5base3.0.Siemens_mn_S7-api_76.pdf

[CIP]		Crisplant Platform

IO 4.0

Transport Protocol

Allen-Bradley CIP Protocol

Journal: io-4.0.p6

[CIPPLC]	Crisplant Platform

Control Logix PLC CIP Interface

Control Logix Mediator Task

Journal: platform.p3_Protocol-CMC-Routing.doc

[CONNECT]	CMC 5

Communication Overview

Journal cmc5base-3.0.s1

[TRANSFER]	Item Transfer Specification

CMC 5 Product Release 3.0

Journal: cmc5base-3.0.s10

[SCANNER]	CMC – Overhead Scanner Interface

RS422

CMC 5 Product Release 3.0

Journal: cmc5base-3.0.p10

[SCALE-B020]	Interface Description, IC <-> Scale

Journal: B020-03E

[SCALE-P30]	CMC – Scale, RS422

Interface Description

Reference: sortercmc.p30

[SCALE-P31]	CMC – Scale, RS422

Interface Description

Reference: sortercmc.p31

[SCALE-P32]	CMC – Scale, RS422

Interface Description

Reference: sortercmc.p32

[VOLUME-P15]	CMC – Volume Scanner, RS422

Interface Description

Reference: sortercmc.p15

[VOLUME-P17]	CMC – Volume Scanner, RS422

Interface Description

Reference: sortercmc.p17

[MES]		Interface Description

CMC Manual Encoding Station (MES) Interface

Reference: sortercommon.p7

[Alibi Messages]	Alibi Messages

Protocol Description

Reference: alibi-sw.p1

## Abbreviations and Acronyms

CMC	Crisplant Machine Controller

CSC	Crisplant System Controller

MAI	Manual Induction Unit

PEC	Photo Electric Cell

RNI	Ready Next Item

SPS	Stray Parcel Supervision

# Transport Protocol

The Routing interface is a telegram (message) based interface. One of the following transport protocols must be used to establish the Routing connection with the CMC. The selection of the transport protocol is dependent of the type of PLC chosen for the CMC.

## Transport Protocols for Siemens S7-1500 PLC Type

When connecting to a Siemens S7-1500 PLC using an external communication processor, one of the following transport protocols may be used:

- RFC1006 (ISOonTCP)

- S7 Functions

When using a Siemens Soft PLC (WinAC-RTX) or a S7-1500 with integrated communication processor, the following transport protocol may be used:

- RFC793 (TCP native)

### RFC 1006

RFC1006 or ISOonTCP as called by Siemens is an open transport protocol for establishing connections between the CSC/Host and the CMC, which is implemented in a S7-1500 Simatic PLC system.

The transport protocol is defined in [RFC1006].

In order to establish an RFC1006 connection with the CMC, a TSAP ID must be used. The following standard TSAP IDs have been defined for the routing interface channel:

| TSAP ID |  |
| --- | --- |
| CMC2HOST_ROUTE_1 | Routing channel from CMC to CSC/HOST (default) |
| HOST2CMC_ROUTE_1 | Routing channel from CSC/HOST to CMC (default) |
| CMC2ST_FBROUTE_1 | Routing channel from CMC to CST |
| ST2CMC_FBROUTE_1 | Routing channel from CST to CMC |

Two connections must be established – one for transmitting and one for receiving telegrams.

### S7 Functions

S7 Functions are a proprietary protocol from Siemens and is available in ‘S7-Softnet’, a commercial product from Siemens. In this product a SAPI (Simple Application Programming Interface) may be used to implement the S7 connections to the CMC.

The S7 Functions must be used in case a S7-1500 H PLC system (Hot standby PLC) is used.

A detailed description of the SAPI interface can be found in [S7-SAPI].

### RFC 793 (TCP Socket)

TCP Socket is a simple protocol to establish connection between CMC and CSC. As opposed to RFC1006 there is no framing of the data stream which calls for an application handling of this issue.

Framing is defined to comprise two parts:

- Start of data

- Length of data

Start of data is two bytes comprising the binary value “0101010101010101” (Most significant bit from left) hex value is 0x5555.

Length of data is one signed integer [0-32767] describing the total length in bytes of the telegram excluding framing data.

In order to establish a RFC793 connection with the CMC a Port number must be used. The following standard Port numbers have been defined for the routing interface channel:

| Port number | Description | TSAP ID (for reference) |
| --- | --- | --- |
| 5031 | Routing channel from CMC to CSC/HOST (default) | CMC2HOST_ROUTE_1 |
| 5030 | Routing channel from CSC/HOST to CMC (default) | HOST2CMC_ROUTE_1 |
| 5034 | Routing channel from CMC to CST | CMC2ST_FBROUTE_1 |
| 5035 | Routing channel from CST to CMC | ST2CMC_FBROUTE_1 |

Two connections must be established – one for transmitting and one for receiving telegrams.

### Keep-alive telegram

Keep alive telegrams are used to ensure that a connection is alive. If a connection has been idle (i.e. no telegrams have been exchanged) for a period of time, a keep-alive telegram is sent to indicate that the connection is still active.

RFC1006

A keep-alive telegram is a dummy telegram with the length of one byte containing the value 0 (zero) – see [RFC1006] for details and timing.

RFC 793

A keep-alive telegram is a dummy telegram comprising the application header with no data payload. The entire telegram looks as this: <5555><0000> (hex)

## Transport Protocols for Allen-Bradley Controllogix PLC Type

When connecting to an Allen Bradley ControlLogix PLC the CIP protocol must be used.

### CIP

CIP is an abbreviation of Control and Information Protocol.

Communication between the CMC and CSC/Host is done using CIP encapsulated in Ethernet/IP explicit messaging using TCP when the CMC is using an Allen-Bradley ControlLogix PLC.

The CIP interface is a memory-based interface. The messages or telegrams are written to memory locations of the PLC and tags identify the memory locations.

For detailed information of the CIP protocol implementation see [CIP] and [CIPPLC].

#### Memory layout for reception of telegrams in PLC

As described in section 3.2.1 the telegrams (or messages) are written to memory locations of the CMC. This section describes the structure of the PLC memory location for reception of telegrams from the CSC/Host.

The data structure for receiving telegrams are defined by a number of segments and each segment consists of a number of bytes where the telegrams are written to by the CSC/Host.

File layout:

“Routing2CMC”

| Segment[0] | Buf[x] of Int (16 bit) |
| --- | --- |
| Segment[1] | Buf[x] of Int (16 bit) |
| Segment[2] | Buf[x] of Int (16 bit) |
| Segment[3] | Buf[x] of Int (16 bit) |
| Segment[4] | Buf[x] of Int (16 bit) |
| ... | ... |
| Segment[Last] | Buf[x] of Int (16 bit) |
| ReadPos |  |

Interface Area: 	“Routing2CMC1”.Segment[X].Buf[Y]

Routing2CMC1:	Tag name for data area for receiving telegrams from CSC/Host. The data area is divided in 100 segments each containing a buffer area of bytes.

Segment:	Data area to hold one telegram block as defined in section 4.1. The size of each segment is defined by the Buf[x] of Int (16 bit) structure. The size of each Buf (segment) array is 225 integers (INT)

ReadPos:	ReadPos is a 16-bit integer defining a pointer to the last segment read by the CMC

The data structure is used as a ring buffer. The CSC/Host will write the telegrams to the next available segment.

Each segment of the ring buffer consists of an integer (16 bit) array with 225 bytes (ControlLogix backplane buffer limit).

Read position is an integer (16 bit) and contains the number of the last segment read by the CMC. The CMC will update the read position each time a telegram block has been handled. The CSC/Host will monitor the read position and make sure not to overwrite segments containing unhandled telegram blocks.

A segment may contain a block of telegrams – see section 4.1.

The mechanism of exchanging telegrams is described in detail in [CIPPLC].

#### Memory layout for reception of telegrams in CSC/host

The PLC used an MSG instruction to send a telegram block to the CSC/Host.

The CSC implementation of the ring buffer is made as a queue. This means that the PLC does not have to inspect the read position before writing.

The CMC does not have to include the message segment TAG in the message, only the content of the message segment.

File layout:

“Routing2Host”

| Segment[0] | Buf[x] of Int (16 bit) |
| --- | --- |

Interface Area: 	“Routing2Host”.Segment[1].Buf[Y]

Routing2Host:	Tag name for data area for receiving telegrams from CSC/Host. The data area is divided in a single segment containing a buffer area of bytes.

Segment:	Data area to hold one telegram block as defined in section ‎4.1 – the size of the segment is defined by the Buf structure. The size of the Buf (segment) array is 225 integers (INT).

#### Byte order and exchange of double integers (DINT)

The byte order is little endian (Allen Bradley standard).

Please note that when double integers (DINT) are written to the buffer, the two integers of the double integer must be swapped. The integer containing the most significant value must be inserted first, then the last integer.

Example of writing a ‘1’ as a double integer:

Buf[x] = 0

Buf[x+1] = 1

#### Framing of telegram blocks in buffer area

As described in [CIPPLC] each telegram block received from the CSC/Host must be framed with the length of the telegram block – see the following drawing:

| Tlg Block length | Header | Telegram 1 | Telegram 2 | Tlg Block length |
| --- | --- | --- | --- | --- |

The block length equals the size of the header (2 integers) and the sizes of the individual telegrams included.

Before the CMC must access the telegram block, the 2 framing parameters must contain the same information.

See section ‎4.1 for specification of the telegram blocks.

#### Keep-alive telegram

Keep alive telegrams are used to ensure that a connection is alive. If a connection has been idle (i.e. no telegrams have been exchanged) for a period of time, a keep-alive telegram is sent to indicate that the connection is still active.

A keep-alive telegram is a dummy telegram with the length of two byte containing the value 0 (zero) – see [CIPPLC] for details and timing.

## Hot Standby Enabling

When the CMC is connected to a hot standby CSC, two routing channels are used – one for Host/CSC System 1 and one for Host/CSC System 2. One functions as master, the other as standby.

The routing connection to the host/CSC with master role is called the master routing connection, the other is called standby routing connection. This means that the role of a specific connection switches as role switches around.

If possible, each routing channel should use a separate CMC network interface and cabling to prevent losing messages because of problems with network card interface or cabling.

The CMC transmits the exact same messages on both channels. Message sequence numbers are identical on the two channels. In case a connection is not established to one of the CSCs, the CMC does not send via that connection.

In a CSC switchover situation, the CMC does not have to stop any sorter devices because one connection to a CSC is always alive. In a switchover situation, the CMC will not get destinations for the items as long as the switchover takes place but the connection to the standby will not disconnect.

To prevent too many items to be stored on the sorter waiting for a destination, it may be a good idea to stop inducting more items until the new master CSC tells the CMC to resume operation. This decision is based on the physical layout of the sorter, recirculation time etc.

If connections are not established to any CSC, the CMC stops sortation because messages from CMC to host/CSC are lost if sortation continues. If the standby has not reported “Host Standby State (active)” to CMC, i.e. not up-and-running, the standby is regarded as if non-connected.

### Routing receive messages

The CMC receives routing telegrams from both the master CSC and standby CSC. From standby it will during normal sortation only react on keep-alive messages, “Host Standby State” and “ResetRequest” messages. In a switchover situation it will receive and react on the messages defined in the switchover flow and then start using that channel as the master channel. Other messages received on the routing channel connected to the hot standby host/CSC are ignored.

### State of standby host/CSC

The CMC must not regard the standby as up-and-running until “Host Standby State (active)” telegram is received. When the standby reports active it is able to take over master role, it is not the master yet.

If CMC detects a timeout on keep-alive messages on the standby connection, it assumes that the standby is inactive, until “Host Standby State (active)” telegram is received again.

State of standby host/CSC is used to determine whether sortation must continue in case the connection to master host/CSC fails.

### Determining master routing channel

After a restart of the CMC, the master routing receive channel is set to be the one where messages (except the “Host Standby State” message) are received. Messages are always sent on both channels.

If a Connected or Reset Request telegram is received, the channel where that telegram is received is the new master channel. After switchover, the CMC now treats the old master channel as a standby channel, refer to section 3.3.1.

### Telegram reception in PLC from CSC/Host System 2

The telegrams (or messages) from CSC/Host system 2 are written to memory locations of the CMC similar to the one described in detail in section 3.2.1.1. The CMC must handle telegrams from CSC/Host system 2 immediately after the handling of telegrams from CSC/Host system 1.

Interface Area: 	“Routing2CMC2”.Segment[X].Buf[Y]

Routing2CMC2:	Tag name for data area for receiving of telegrams from CSC/Host system 2.

# Application Protocol

The application protocol describes the actual telegram information flow between the CMC and the CSC/Host.

The application protocol is independent of the transport protocol used.

## Routing Application Telegrams

In order to optimize communication between the CMC and a CSC or an external Host, the telegrams must be exchanged in telegram blocks. A telegram block consists of a header followed by a number of telegrams. The CMC will only handle one telegram block in each scan.

In case the CSC/Host must send a number of telegrams (e.g. Carrier Data Reboot), these telegrams must be included in a number of telegram blocks (max. 80-100 words of telegrams in each telegram block).

The buffering of data must be done in 50-100 milliseconds if the telegrams concern item information – e.g. redirection of an item on the sorter. In other cases, the buffering may be done over a longer period if the telegrams concerned do not concern items.

The telegram block containing more telegrams has the following format:

| Header | Telegram 1 | Telegram 2 | Telegram 3 ... |
| --- | --- | --- | --- |

The header contains the identification of the channel and the version of the protocol.

The protocol version identifies the current version of the protocol. This version number enables the CSC or external host to load the proper protocol handler.

Protocol version 3:

The header consists of 2 words for routing protocol.

| Channel ID | Protocol Version |
| --- | --- |

Protocol version 4:

The header consists of 4 words for routing protocol. In this protocol version a telegram block sequence number is added to the header.

| Channel ID | Protocol Version | Sequence Number (2 words) |
| --- | --- | --- |

All telegrams begin with a header of 2/4 words, a 1 word "request code" that identifies the telegram and the telegram length:

| Request Code | Telegram Length | Telegram Data 1 | Telegram Data 2 ... |
| --- | --- | --- | --- |

The telegram length includes all data in the telegram including the Request Code.

The telegram contents consist of integer and double integer numbers where nothing else is specified.

For an example of telegram blocks - see section 5.1.2.

### Sequence number (Protocol version 4 only)

The sequence number is two words (32 bit) wide. Sequence numbers on transmit and receive telegrams blocks are independent.

The transmitter increments the sequence number for each telegram block it sends. When the number reaches the max limit (2,147,483,647), it wraps down to 1.

Sequence number 0 is reserved for initialization of communication. If a communication partner forgets which sequence number to use next for a transmission (e.g. when doing a reset of the CMC), it is allowed to use the value 0. The receiver should always accept sequence number 0. When a connection is established, sequence number is not set to 0.

In CMC hot standby systems, the CMC uses the received sequence number to detect doublets since each CMC is connected to a CSC using 2 channels (one on each network interface), i.e. 4 channels in total. To prevent the CMC from discarding messages as doublets after a switchover, the CMC should accept the sequence number in "Connected" telegram and then start detecting doublets again.

When it is not a hot-standby CMC that is used, the CMC does not check the received sequence number.

## Routing

The Routing channel is used for controlling the items on the sorter.

The routing telegrams numbers are divided in the following groups:

| Telegram No. |  |
| --- | --- |
| 00 - 09 | Initialisation telegrams |
| 30 - 39 | Chute control |
| 40 - 59 | Item routing control |
| 60 - 69 | Carrier information |
| 70 - 79 | Input/Output and Function control |
| 90 - 99 | Item and dump chute configuration; Status |
| 100 - 109 | Miscellaneous telegrams |
| 110 - | Project-specific telegrams |

CSC Mode or Host Mode

The CMC may be set up in 2 modes, each of which influences the telegram flow between the CMC and CSC or a Host system.

In CSC mode the CMC is not allowed to redirect items to dump chutes on its own – the items will always have the destination as assigned by the CSC. Furthermore, the CSC will request carrier data when performing a hard reset.

In Host Mode the CMC is allowed to redirect items to a dump chute when the condition to do so is met – e.g. redirect to Max. recirculation dump chute when an item has recirculated the max. recirculation times. Furthermore, the CMC may be configured to request carrier data when performing a hard reset. This may only be done if the Host contains all carrier data.

Response time requirement

It is essential that the CSC/Host responses to a request for destinations (Scanner/Keyboard result) as fast as possible – any delay in the response may cause an item to recirculate on the sorter thus delaying the sortation of the item and thereby reducing the overall capacity of the sorter.

The theoretical maximum allowed response time depends on the sorter layout – e.g. the distance from an overhead scanner and the first chute after the overhead scanner; the scanner type selected – and the speed of the sorter.

In the light of the above the maximum response time from the CSC/Host may not exceed 500 mS.

CMC index and Item ID number

Items are identified by index numbers. The CMC will assign the index number to an item and this index number is unique as long as the item is on the sorter. Afterwards the index may be reused. In order to let the CSC or an external host uniquely identify an item or group of items before and after an item has been sorted by the CMC an Item ID number may be attached to the item.

The Item ID number is a double integer number and is in the range from 0 to 2.147.483.647.

The CSC or external host may send the Item ID to the CMC when a CMC index has been assigned to an item. The CMC will save the Item ID number and whenever a telegram with the CMC index is sent to the CSC or external host the Item ID number will be attached to the telegram.

The CMC does not check whether the same Item ID is already in use on the sorter or at an induction unit. If the CMC has not received an Item ID from the CSC/Host, the Item ID number is 0.

In dispatch mode the CMC assigns the items Item ID 0. The CSC or external host may update the Item ID number at any time.

An SPS/MAI without items in the code queue will work like dispatch mode.

Sorter part numbering

The sorter parts are numbered as follows:

Induction Units:	0..99

SPS/MAI:		100..199

Chute Destination:	0 .. 2999

Item parked on Sorter:	3000..3089

CMC Item Parked on sorter:	3090-3099

Overhead Scanner:	4000..4098

Any Overhead Scanner:	4099

Carrier:		1..3999

Item Index:		0..4999

The total numbering (e.g. Induction unit: 0..99) is not an expression for how many devices of a certain sorter part the CMC can handle by default – it only describes the addressing area of the sorter parts in question.

In the following chapters a description of the telegrams is organized according to their “request code” representation.

The following table shows the telegrams defined for the Routing channel.

| Telegram no.: | Telegram name | Origin | Option | Host interface | DHL |
| --- | --- | --- | --- | --- | --- |
| 00 | Reset Request | CSC/Host |  | * | x |
| 01 | Reset Completed | CMC |  | * | x |
| 05 | Ready | CMC+Host |  | * | x |
| 10 | Code | CSC/Host |  | (1) | x |
| 11 | Set Induction Unit Mode | CSC/Host |  | (2) |  |
| 12 | Code Acknowledgement | CMC |  | (1) | x |
| 13 | Negative Acknowledgement | CMC |  | (1) | x |
| 14 | Item ID | CSC/Host |  | (3) | x |
| 15 | Clear Queue | CSC/Host |  | (1) |  |
| 16 | Queue Cleared | CMC |  | (1) |  |
| 17 | Code Not Used | CMC |  | * | x |
| 18 | Item ID Acknowledgement | CMC |  | (3) |  |
| 19 | Code Used | CMC |  | * | x |
| 20 | Set Induction Unit Mode Acknowledge | CMC |  | (2) |  |
| 21 | Code-MultiDest | CSC/Host | Option |  | x |
| 22 | Item Transfer | CMC | Option |  |  |
| 23 | MES Code Error | MES (CMC) | Option | (6) |  |
| 24 | Code Request | MES (CMC) | Option | (6) |  |
| 25 | Code Result | CSC/Host | Option | (6) |  |
| 26 | Scanner/Keyboard Result | CMC |  | (1+4) | P18+ |
| 27 | Scale Result | CMC |  | (7) | P18+ |
| 28 | Volume Result | CMC |  |  | P18+ |
| 29 | Item Arrived | CMC | Option |  |  |
| 30 | Chute Status | CMC |  | * | x |
| 31 | Chute Status Request | CSC/Host |  |  | x |
| 32 | Block Chute | CSC/Host |  |  | x |
| 35 | Start/Stop Sorter | CSC/Host | Option |  |  |
| 36 | Start/Stop Induction Units | CSC/Host | Option |  |  |
| 37 | Start/Stop Sorter Acknowledge | CMC | Option |  |  |
| 38 | Start/Stop Induction Unit Acknowledge | CMC | Option |  |  |
| 40 | Item Inducted | CMC |  | * | x |
| 41 | Item Redirect | CSC/Host |  | * | x |
| 42 | Item Redirect Acknowledgement | CMC |  | * | x |
| 43 | Item Discharged | CMC |  | * | x |
| 44 | Discharge Supervision | CMC |  | * | x |
| 45 | Item Sortation Event | CMC |  | * | x |
| 46 | Item Disappeared | CSC/Host | Option |  |  |
| 47 | Item Redirect–MultiDest | CSC/Host | Option |  |  |
| 60 | Carrier Data Reboot | CMC |  | (5) |  |
| 61 | Carrier Data Reboot Ack. | CMC |  | (5) |  |
| 63 | Request Carrier Data Reboot | CSC/Host |  | (5) |  |
| 64 | Carrier Data Transmitted | CSC/Host |  | (5) |  |
| 65 | Index Status | CMC |  |  |  |
| 66 | Request Index Status | CMC |  |  |  |
| 70 | Input Changed | CSC/Host | Option |  |  |
| 73 | Set Function | CSC/Host | Option |  | x |
| 74 | Function Status | CMC | Option |  | x |
| 75 | Set Output | CMC | Option |  |  |
| 90 | Req Max. Recirculation Count | CMC |  |  |  |
| 91 | Max. Recirculation | CMC |  |  |  |
| 95 | Dump Chutes | CMC |  |  |  |
| 96 | Request Dump Chutes | CMC |  |  |  |
| 97 | Set Status | CMC |  |  |  |
| 99 | Status | CMC |  | * | x |
| 100 | Reject Telegram | CMC |  | * | x |

The ‘Minimal Host Interface’ column describes the telegram an external host, as a minimum must implement to control the communication and items of the sorter. The figures in () in this column has the following meaning:

(*)	Mandatory telegrams – must always be implemented.

(1)		These telegrams must be implemented if keyboards are used at one or more induction units – or more correct if the induction unit must run in Keyboard mode.

(2)	These telegrams must be implemented if the induction unit mode of the induction unit must be controlled from the Host.

(3)	These telegrams allow the Host to assign a unique identification to an item.

(4)	This telegram must be implemented if one or more overhead scanners are used at the induction units or the sorter.

(5)	These telegrams must be implemented if the Host contains the carrier table of the CMC. The data are transferred during a reset of the CMC.

(6)	The telegrams are used when a MES (keyboard) is attached directly to the CMC.

(7)	The telegram supports both the Eilertsen scale and OCS scales. Some of the sections in the telegram will mean different things depending on the scales being used.

Certified weighing and volume scanning

Only the “Scale Result” (27) and “Volume Result” (28) telegrams are a part of the

ALIBI MESSAGES PROTOCOL DESCRIPTION (p1_Protocol-AlibiMessages.doc) [ALIBI].

### Reset request (00)

CSC/Host => CMC

Format:	TTLL_X

TT	0 = Telegram Type

LL	3 = Telegram Length

X	0 = Soft reset.

1 = Hard reset.

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | X |

Request for a CMC-reset.

Generally

The sorter has to be stopped for reset and if it has not been stopped before sending the reset request telegram, the sorter will stop immediately and uncontrolled, due to stop of program execution when it receives the telegram.

After reset, the CMC will answer with a Reset Completed (01) telegram.

The CSC/Host must always send a Reset Request when the Routing connection is established in order to synchronise the states of the two parties.

Hard reset

Hard reset reinitializes program and data storage in the CMC, including the item carrier data storage. The CMC will have no knowledge of items on the sorter after a hard reset. Furthermore, the CMC synchronisation with the sorter chain is removed. This means that the CMC will start looking for the synchronisation point when starting the sorter again.

A hard reset will also force the CMC to recalculate the configuration parameters. This means that in order to activate changes in distance for e.g. chutes, a hard reset of the CMC must be performed.

If the CMC is set up for it, the CMC will request carrier data in connection with a hard reset (CSC mode).

The hard reset enables all carriers and clears all information of carrier faults.

Soft reset

A soft reset reinitializes program and data storage in the CMC storage. The CMC synchronisation with the sorter chain is kept and the carrier data is kept.

No recalculation of the configuration parameters is done during a soft reset.

### Reset completed (01)

CMC => CSC/Host

Channel: Routing

Format:	TTLL_X

TT	1 = Telegram Type

LL	3 = Telegram Length

X	0 = Soft reset

1 = Hard reset

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | X |

CMC reset is complete.

The telegram will contain information of the type of reset performed (see section ‎4.2.1 for details).

### Connected (02)

CMC => CSC/Host

CSC/Host => CMC

Channel: Routing

Format:	TTLL

TT	2 = Telegram Type

LL	2 = Telegram Length

File layout:

Type

Length

CSC and CMC reports Connected in a hot standby switch over.

For a scenario of the hot standby switch over, see section 5.2.1.

### Hot standby state (03)

CSC/Host => CMC

Channel: Routing

Format:	TTLL_X

TT	3 = Telegram Type

LL	3 = Telegram Length

X	0 = Non Active

1 = Active

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | X |

Hot standby Host/CSC reports if it is active and able to take over control.

If CMC has a timeout on keep-alive messages on the standby connections, it assumes that the standby is non-active, until the “Host Standby State (active)” telegram is received again.

For a scenario of the hot standby switch over, see section ‎5.2.2.

### Ready (05)

CMC => CSC/Host

CSC/Host => CMC

Channel: Routing

Format:	TTLL

TT	5 = Telegram Type

LL	2 = Telegram Length

File layout:

Type

Length

After initialization, the CMC sends a ready telegram to the CSC/Host and the CSC/Host also sends a ready telegram to the CMC to inform that the CSC/Host is ready to accept sorting information.

The sorter is not able to start before the CMC has sent a Ready telegram and the CMC has received a Ready telegram from the CSC/Host.

Chute status telegrams (30) for all chutes are sent after Ready telegram.

For a scenario of the Reset sequence see section 5.2.1.

### Resume (06)

CMC => CSC/Host

CSC/Host => CMC

Channel: Routing

Format:	TTLL

TT	6 = Telegram Type

LL	2 = Telegram Length

File layout:

Type

Length

In a hot standby switch over scenario, the CMC or CSC/host sends a Resume when ready to resume operation.

For a scenario of the hot standby switch over see section ‎5.2.1.

### Request host standby state (07)

CMC => CSC/Host

Channel: Routing

Format:	TTLL_X

TT	7 = Telegram Type

LL	2 = Telegram Length

File layout:

Type

Length

CMC requests Hot standby Host/CSC for info regarding if it is active and able to take over control. Replied by “Host Standby State” telegram.

If CMC has a timeout on keep-alive messages on the standby connections, it assumes that the standby is non-active, until the “Host Standby State (active)” telegram is received again. When keep alive is received again, the CMC requests “Host Standby State” by using this telegram.

For a scenario of the hot standby switch over, see section ‎5.2.2.

### Code (10)

CSC/Host => CMC

Channel: Routing

Format:	TTLL_IDDGGGG

TT	10 = Telegram Type

LL	6 = Telegram Length

I	Induction unit number	(0..199)

DD	Destination number	(0..4199)

0..2999	= Chute 0-2999

3000..3089	= Item parked on sorter

4000..4098	= Scanner number 0-98

4099		= Any scanner

4100..4198	= Volume scanner No. 0-98

4199		= Any volume scanner

GGGG	Item ID number	(0..2.147.483.647)

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | I |
| D | D |
| G | G |
| G | G |

Before an item is allowed to enter an induction unit (at the "code-PEC"), the CSC/Host must send a code telegram (10) to the CMC.

The CSC/Host must send no further code telegrams to the selected induction unit until the CMC has acknowledged receipt with a code acknowledgement (12) or a negative code acknowledgement telegram (13).

The Item ID number is attached to the Index number when the telegram is received and acknowledged. The CMC does not check if the Item ID number is already used.

### Set induction unit mode (11)

CSC/Host => CMC

Channel: Routing

Format:	TTLL_I_MDD

TT	11 = Telegram Type

LL	5 = Telegram Length

I	Induction unit number	(0..199)

M	Mode	(0..1)

0	= Keyboard mode

1	= Dispatch mode

DD	Destination	(0..4199)

0..2999	= Chute destination

3000..3089	= Item parked on sorter

4000..4098	= Overhead scanner

4099		= Any scanner

4100..4198	= Volume scanner No. 0-98

4199		= Any volume scanner

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | I |
|  | M |
| D | D |

The Set Induction Unit Mode telegram is used to control the mode of an induction unit.

In keyboard mode all items arriving at the induction unit (item detected by the Code PEC) must have a destination in order to enter the sorter. This destination must be assigned to an item with a Code telegram (see section ‎4.2.8)

In Dispatch mode all items arriving at the induction unit will be inducted to the same destination e.g. an overhead scanner. The destination is defined by this telegram (Set Induction Unit Mode telegram). The scanner mode defined in a CSC is the same as the dispatch mode with a scanner destination.

The CSC/Host must send no further telegrams to the selected induction unit until the CMC has acknowledged receipt with a Set Induction Unit Mode telegram (20).

After acknowledgement of this telegram the corresponding induction unit remains in "Dispatch Mode" or “Keyboard mode” until the CMC receives a new Set Induction Unit Mode (11) with another mode.

If the induction unit is already in dispatch mode when a new Set Induction Unit Mode with dispatch mode, the telegram is accepted and the destination from the telegram is used as the new destination.

If the induction unit is already in keyboard mode when a new Set Induction Unit Mode with keyboard mode specified is received the telegram is accepted – but no action is performed by the CMC.

If the mode is changed from keyboard to dispatch and there are indexes for items in the associated code queue, ‘Code Not Used’ telegrams (17) with reason 5 (Clearing Code Queue) are sent for each code to the CSC/Host.

The Item ID number is 0 for all items in dispatch mode until an item ID telegram (14) is received from the CSC/Host.

### Code acknowledgement (12)

CMC => CSC/Host

Channel: Routing

Format:	TTLL_IXXGGGG

TT	12 = Telegram Type

LL	7 = Telegram Length

I	Induction unit number	(0..199)

XX	Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

DD	Destination number	(0..4199)

0..2999 	= Chute 0-2999

3000..3089	= Item parked on sorter

3090..3099	= CMC Item parked on sorter

4000..4098 	= Scanner number 0-98

4099 	= Any scanner

4100..4198	= Volume scanner No. 0-98

4199		= Any volume scanner

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | I |
| X | X |
| G | G |
| G | G |
| D | D |

After the CMC has received a code telegram (10) or a code-multidest telegram (21), the contents of the telegram will be checked and a code acknowledgement telegram (12) or a code negative acknowledgement telegram (13) is sent to the CSC/Host.

If the telegram contents are accepted, a code acknowledgement telegram (12) is sent.

If the telegram contents are invalid, a code negative acknowledgement telegram (13) will be sent.

If the code queue for the corresponding induction unit is full, a code negative acknowledgement telegram (13) will be sent.

Max. queue length is 5.

### Negative acknowledgement (13)

CMC => CSC/Host

Channel: Routing

Format:	TTLL_P_IGGGG

TT	13 = Telegram Type

LL	6 = Telegram Length

P	Reason	(1..7)

1 =	Induction unit unknown

2 =	Destination unknown

3 =	Scanner unknown

4 =	Queue full

5 =	Item Dispatch mode

6 = Wrong sorter side (Double tray only)

7 = Mixed Destinations

I		Induction unit number	(0..199)

GGGG	Item ID number	(0..2.147.483.647)

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | P |
|  | I |
| G | G |
| G | G |

After the CMC has received a code telegram (10) or a code-multidest telegram (21), the contents of the telegram will be checked and a code acknowledgement telegram (12) or a code negative acknowledgement telegram (13) is sent to the CSC/Host.

The reason codes have the following meaning:

| Reason code | Description |
| --- | --- |
| Induction unit unknown | The induction unit identified by ‘Induction unit number’ in the Code telegram (10), Set Induction Unit Mode telegram (11) or Code-MultiDest (21)) is unknown |
| Destination unknown | The destination (chute) identified by ‘Destination number’ in the Code telegram (10), Set Induction Unit Mode telegram (11) or Code-MultiDest (21) is unknown |
| Scanner unknown | The scanner identified by ‘Destination number’ in the Code telegram (10), Set Induction Unit Mode telegram (11) or Code-MultiDest (21) is unknown |
| Queue full | The Code queue is full and cannot contain the code – the code received in the Code telegram (10) is ignored. |
| Item Dispatch mode | The induction unit is in Dispatch mode – the Code is ignored. |
| Wrong side of sorter | The destination received in the Code telegram (10), Set Induction Unit Mode telegram (11) or Code-MultiDest (21) cannot be reached from the induction unit in question (double tray sorter only) |
| Mixed Destinations | If more than 1 destination is received in the Code-MultiDest telegram (21), the destinations must all be of the type ‘Chute’. Otherwise the code is rejected with this reason. |

Max. queue length is 5.

The originally Code telegram is ignored in case a Negative Acknowledgement has to be sent.

### Item ID (14)

CSC/Host => CMC

Channel: Routing

Format:	TTLLXXGGGG

TT	14 = Telegram Type

LL	5 = Telegram Length

XX	Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

File layout:

| Type |  |
| --- | --- |
| Length |  |
| X | X |
| G | G |
| G | G |

When the CSC/Host sends this telegram, the CMC attaches the Item ID number to the Index number. The CMC overwrites the original Item ID number. The CMC does not check whether the Item ID number is already used. Every time the CMC sends a telegram with index number, the Item ID number is included as well.

The CMC sends an item ID acknowledgement telegram (18) as an answer to this telegram.

### Clear queue (15)

CSC/Host => CMC

Channel: Routing

Format:	TTLL_N_I

TT	15 = Telegram Type

LL	4 = Telegram Length

N	Number of codes to clear	(0..5)

I	Induction unit number		(0..199)

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | N |
|  | I |

Request to clear a number of codes not yet used by a code used telegram from the item queue in the CMC. The last code sent to the queue is cleared first.

If the number of codes to clear is higher than the number of codes in the queue, the telegram is accepted and all codes in the queue are cleared.

If the number of codes to clear is 0, no codes are cleared, but a queue cleared telegram (16) is sent containing the number of codes in the queue.

This telegram will always be answered by a queue-cleared telegram (16).

If the induction unit number is unknown, the induction unit number in the queue-cleared telegram is 99.

For each code cleared in the queue, a Code Not Used telegram with reason 5 (Clearing Code Queue) is sent to the CSC/Host – see example in section ‎5.2.8.

### Queue cleared (16)

CMC => CSC/Host

Channel: Routing

Format:	TTLL_N_I

TT	16 = Telegram Type

LL	4 = Telegram Length

N	Number of remaining codes in queue	(0..5)

I	Induction unit number				(0..199)

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | N |
|  | I |

The CMC sends a queue-cleared telegram after receipt of a clear queue telegram (15).

If the number of codes to clear was higher than the number of codes in the queue, all codes in the queue were cleared.

If an unknown Induction unit number in Clear Queue telegram => I = 255

### Code not used (17)

CMC => CSC/Host

Channel: Routing

Format:	TTLL_S_IXXGGGG_B

TT	13 = Telegram Type

LL	8 = Telegram Length

S	Reason	0 = Induction Unit Error

1 = Item Disappeared

2 = Item Oversize

3 = Item Overweight

4 = Item on Disabled Carrier

5 = Clearing Code Queue

I	Induction unit number	(0..199)

XX	Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

B	Belt number	(0..15)

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | S |
|  | I |
| X | X |
| G | G |
| G | G |
|  | B |

When the CMC, on an induction unit, needs to cancel an index on which telegram "Code Used" already has been sent, it has to send a "Code Not Used" telegram to the CSC/Host

To give the CSC/Host opportunity to trace why items (indexes) are cancelled, the "Code Not Used" telegram holds information about the reason why an item (index) was cancelled.

If an item (index) is cancelled, but not due to an error on the item (index) itself, but due to an error on the induction unit or due to an error on another item on the induction unit, a reason 0 is used.

If an item (index) is cancelled due to an item error on the item (index) itself, either a reason 1, 2, or 3 is used.

An item error is defined as an error on the specific item, which does not affect other items on the induction unit, meaning the other items will be inducted after the erroneous item has been removed (if not disappeared)

Reason 4 (Item on disabled carrier) is used when an MAI with a code in the code queue detects a new item on a disabled carrier. A Code Used telegram followed by a CodeNotUsed telegram will be sent.

Reason 5 is used when clearing the code queue. For each code in the code queue a CodeNotUsed telegram is sent.

### Item ID acknowledgement (18)

CMC => CSC/Host

Channel: Routing

Format:	TTLLXXGGGGCC_S

TT	18 = Telegram Type

LL	7 = Telegram Length

XX	Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

CC	Carrier number	(0..3999)

(0) 			Item at induction unit

(1-3999)	Carrier number

S	Reason	0 = OK

1 = Index error

File layout:

| Type |  |
| --- | --- |
| Length |  |
| X | X |
| G | G |
| G | G |
| C | C |
|  | S |

When the CMC has received an item ID telegram, the index number in the telegram is checked and an item ID acknowledgement telegram is sent to the CSC/Host.

If the index number is accepted, an item ID acknowledgement telegram with reason 0 (OK) is sent.

If the index number is not in use, an item ID acknowledgement telegram with a reason 1 (not OK) is sent.

### Code used (19)

CMC => CSC/Host

Channel: Routing

Format:	TTLL_IXXGGGG

TT	19 = Telegram Type

LL	6 = Telegram Length

I	Induction unit number	(0..199)

XX	Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | I |
| X | X |
| G | G |
| G | G |

The CMC sends a code used telegram at the moment a code is used from the code queue.

Code used is not sent for stray items.

If no item ID is available, the value is 0. Then the CSC/Host can send an item ID telegram (14) to give the item another Item ID number.

### Induction unit mode status (20)

CMC => CSC/Host

Channel: Routing

Format :	TTLL_I_R_MDD

TT	20 = Telegram Type

LL	6 = Telegram Length

I	Induction unit number	(0..199)

R	Reason	(0..6)

0	= Accepted

1	= Induction unit unknown

2	= Destination unknown

3	= Scanner unknown

6	= Wrong side of sorter (db. tray only)

M	Mode	(0..1)

0	= Keyboard mode

1	= Dispatch mode

2	= Cyclic Mode

DD	Destination	(0..4199)

0..2999	= Chute destination

3000..3089	= Item parked on sorter

4000..4098	= Overhead scanner

4099		= Any scanner

4100..4198	= Volume scanner No. 0-98

4199		= Any volume scanner

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | I |
|  | R |
|  | M |
| D | D |

This telegram is sent on the Routing channel whenever the mode of an induction unit changes e.g. when the mode is changed through the Command & Control channel. This means that the telegram may be sent on the Routing channel spontaneously.

The Mode will reflect the new mode of the induction unit and if the mode of the induction unit is Dispatch mode the destination parameter will contain the dispatch destination.

Cyclic mode is a test mode where the induction unit distributes the item inducted to a section or all chutes on the sorter.

The telegram is also used as an acknowledge of the Set Induction Unit Mode telegram (11). If the mode is accepted a Reason 0 is returned. Otherwise an error reason is sent, and the induction unit remains in the same mode as before receiving the Set Induction Unit Mode telegram.

### Code – multi dest (21)

CSC/Host => CMC

Channel: Routing

Format:	TTLL_IGGGG_SPP_NAABBCCDDEE

TT	21 = Telegram Type

LL	13 = Telegram Length

I	Induction Unit Number	(0..199)

GGGG	Item ID Number	(0..2.147.483.647)

S	Mode	0 = Normal

1 = Nearest Destination

PP	Destination Number	(0..4199)

0..2999	= Chute 0-2999

3000..3089	= Item parked on sorter

4000..4098 	= Scanner number 0-98

4099 	= Any scanner

4100..4198	= Volume scanner No. 0-98

4199		= Any volume scanner

N	No. of Alt. Destinations	(0..5)

AA	Alternative Destination 1	(0..2999) 	= Chute 0-2999

BB	Alternative Destination 2	(0..2999) 	= Chute 0-2999

CC	Alternative Destination 3	(0..2999) 	= Chute 0-2999

DD	Alternative Destination 4	(0..2999) 	= Chute 0-2999

EE	Alternative Destination 5	(0..2999) 	= Chute 0-2999

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | I |
| G | G |
| G | G |
|  | S |
| P | P |
|  | N |
| A | A |
| B | B |
| C | C |
| D | D |
| E | E |

Before an item is allowed to enter an induction unit (at the "code-PEC"), the CSC/Host must send either a ‘Code’ telegram (10) or a ‘Code-Multi Dest’ telegram (21) to the CMC.

The CSC/Host must not send any further code telegrams to the selected induction unit until the CMC has acknowledged receipt with a code acknowledgement (12) or a negative code acknowledgement telegram (13).

The Item ID number is attached to the Index number when the telegram is received and acknowledged. The CMC does not check whether the Item ID number is already used.

Only destinations of same type must be used in the telegram – i.e. if a scanner and a destination are included in the telegram the ‘Code Multi-Dest’ telegram is rejected.

Only chute destinations are allowed in the alternative destinations 1-5.

The field ‘No. of alternative destinations’ specifies how many valid alternative destinations (up to 5) are included in the telegram – i.e. if the number 2 is specified, the alternative destinations 3, 4 and 5 are not used or checked by the CMC.

The destination used in all other telegrams will always be the current destination of the item – i.e. when the item is inducted the destination chosen for the item is used.

The ‘Mode’ field has the following meaning:

| Normal: | The Destination in ‘PP’ will be used as the first destination of the item. If the item cannot be sorted to this destination, the first alternative destination (specified in the field ‘AA’) will be the next destination for the item and so on. When the CMC has tried the alternative destination 5 without success, the original destination will be used again. The CMC will try to sort the item switching between the destinations in turn until the item has recirculated ‘Max. recirculation’ times – after this the item is sorted to the Max. Recirculation dump chute. |
| --- | --- |
| Nearest destination: | The CMC will evaluate which of the specified destinations is nearest to the induction unit and induct the item to this destination. If the item cannot be sorted to this destination, the CMC will use the next destination specified in the telegram – e.g. if the ‘Alternative destination 2’ was used the next destination will be ‘Alternative destination 3’. When the mode ‘Nearest Destination’ is used, the destinations must be listed in numerical order – i.e. the numerical smallest destination number in the field ‘Destination’ (‘PP’) and the numerical highest number in the field ‘Alternative Destination 5’ (‘EE’). |

### Item transfer (22) (option)

CMC => CSC

Format:	TTLL_UNNXXDD_H

TT	22 = Telegram Type

LL	 9 = Telegram Length

U	Unit type	0	= Induction unit

1 	= Chute

NN	Unit number	(0..2999)

XX	Index number	(0..4999)

GGGG	Item ID Number	(0..2.147.483.647)

DD	Destination	(0..3999)

H	Handshake Number	(0..7 or 8)

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | U |
| N | N |
| X | X |
| G | G |
| G | G |
| D | D |
|  | H |

The Transfer Item telegram is used to transfer information of digital handshake numbers to the CSC. This allows the CSC to track an item between a conveyor system and the sorter and vice versa.

At an induction unit an item is received from a conveyor system and at a chute the CMC transfers an item to a conveyor system.

If the transfer is at an induction unit, the Unit Type is set to Induction unit and the Unit number will identify the induction unit. The Item Transfer telegram is in this case the first telegram received for the item.

If the transfer is at a chute, the Unit Type is set to Chute and the Unit number will identify the chute. The Item Transfer telegram is in this case received after Item Discharged telegram and before Discharge Supervision telegram.

If the CSC/Host has set the Item ID number for the item, this number is included – otherwise the Item ID will be set to 0 (zero).

The destination will contain the current destination of the item. If the item is being transferred at a chute, the unit number and destination number will be the same.

This transfer is using RNI/NIP digital signals too – see the project-specific interface document [TRANSFER] for timing issues etc. RNI is an abbreviation for Ready Next Item and this signal is controlled by the receiving system. NIP is an abbreviation for Next Item Proceeding and the signal is controlled by the originating system.

If the handshake number is set to ‘8’, it means that the induction unit has seen an item but no NIP was received. The CMC must then handle the item as a stray.

### MES code error (23) (option)

MES => (CMC) => Host

Channel: Routing

Format:	TTLL_P_IGGGG

TT	23 = Telegram Type

LL	5 = Telegram Length

I	Induction Unit number	(0-199)

SS	Sequence Number	Sequence number generated by MES

P	Reason	(1..2)

1	= Timeout

2	= Out of sequence

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | I |
| S | S |
|  | P |

This telegram is sent from the MES in case the interface between the MES and Host is jeopardized.

The MES sends the telegram to the CMC, which forwards the telegram unchanged to the Host.

The sequence number identifies the corresponding Code Request (see section ‎4.2.22).

A reason code is included, and the reason codes have the following meaning:

| Timeout | No ‘Code Result’ telegram has been received by the MES within a certain time limit. |
| --- | --- |
| Out of sequence | If a Code Result telegram is received out of sequence, the MES will send a MES Code Error telegram with this reason to the CMC. |

The original Code Request telegram is ignored in case a MES Code Error telegram has to be sent.

For telegram scenarios and other information please consult the document [MES].

### Code request (24) (option)

MES => (CMC) => CSC/Host

Format:	TTLLUU_I_M_K_La1a2....all-1all

TT	24 =Telegram Type

LL	6 + (L+1)/2 = Telegram Length

I	Induction Unit number	(0-199)

SS	Sequence Number	Sequence number generated by MES

K	Key	Source of code or key pressed

0	= Scanner

1	= Keyboard

10	= Special key 0

11	= Special key 1

L	Code length	Number of ASCII characters in response

a1a2....all-1all Code	Code in ASCII (0..127)

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | I |
| S | S |
|  | K |
|  | L |
| a1 | a2 |
| a(len-1) | a(len) |

When a code is entered at the MES, a Code Request is sent to the CMC from the MES. The CMC will transfer the telegram to the CSC/Host.

The sequence number is controlled by the MES and is used to distinguish between more Code Requests – or more precisely, to uniquely identify the corresponding Code Result telegram. The CSC/Host must return the sequence number in the Code Result telegram.

Key indicates the source of the code or if a special key has been pressed at the MES.

If the code length is uneven, the last byte is set to 0.

The content of the code field must correspond with the format described in [SCANNER] document. The format supports multi barcodes/key string entries.

The content of the telegram is not checked by the CMC – the telegram is transferred to the CSC/Host unconditionally.

For telegram scenarios etc. please consult [MES].

### Code result (25) (option)

CSC/Host => (CMC) => MES

Format:	TTLLDDXX_H_C_La1a2....all-1all

TT	25 = Telegram Type

LL	13 + (L1+1)/2 + (L2+1)/2 + (L3+1)/2 + (L4+1)/2 = Telegram Length

I	Induction Unit number		(0-199)

SS	Sequence Number		Sequence from corresponding Code Request telegram

GGGG	Item ID number		(0..2.147.483.647)

DD	Destination		Destination item is sent to

H	HdlRes		Code Handling Result

0 = OK

1 = Timeout in code conversion

2 = Code too long

10 = CMC Code Nack

C	ConvRes		Code Convert Result

0 = OK

1 = res 1

2 = res 2

L1	Barcode length		Number of ASCII characters in the barcode

((C || H) < > 0  => L = 0).

a1a2....all-1all barcode		Barcode in ASCII (0..127)

L2	Code Conv String length	Number of ASCII characters in the string used to code convert

((C || H) < > 0  => L = 0).

b1 b2....bll-1bll	 Code conv string		String in ASCII (0..127)

L3	Conv res String length		[0..30]

Number of ASCII characters in conv. res. result string

((C || H) < > 0  => L = 0).

c1c2....cll-1cll Conv res String		Conversion result in ASCII (0..127)

L4	Display Name String length	[0..15]

Number of ASCII characters in Display name string

((C || H) < > 0  => L = 0).

d1d2....dll-1dll Display Name String		Destination name in ASCII (0..127)

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | I |
| S | S |
| G | G |
| G | G |
| D | D |
|  | H |
|  | C |
| L1 | L1 |
| a1 | a2 |
| a(len-1) | a(len) |
| L2 | L2 |
| b1 | b2 |
| b(len-1) | b(len) |
| L3 | L3 |
| c1 | c2 |
| c(len-1) | c(len) |
| L4 | L4 |
| d1 | d2 |
| d(len-1) | d(len) |

The content of the telegram is not checked by the CMC – the telegram is transferred to the CSC/Host unconditionally.

The sequence number is copied from the corresponding Code Request telegram. The sequence number is used to uniquely identify a Code Request and Code Result pair.

ConvRes is used for code convert result codes whereas HdlRes is used for handling result codes, e.g. code too long, timeout etc.

The barcode field must contain the same barcode as the CSC/Host received in the corresponding Code Request telegram.

The ‘Code conv string’ field contains the part of the barcode which was used for the code conversion.

The ‘Conv res string’ field contains the code conversion result as an ASCII text string, e.g. “OK”, “Unknown code” etc.

Destination display name is included for display purposes at the MES.

In case the CMC cannot forward the Code Request belonging to the Code Result (e.g. the connection to the Host is down), the CMC will forward a Code Result telegram with the status ‘CMC Code Nack’.

For telegram scenarios etc. please consult [MES].

### Scanner result (26)

CMC => CSC/Host

Channel: Routing

Format:	TTLLUUXXGGGGCC_M_S_La1a2....all-1all

TT	26 = Telegram Type

LL	10 + (L+1)/2 = Telegram Length

UU	Unit number	(4000-4099)	Scanner number

XX	Index number	(0-4999)

GGGG	Item ID number	(0..2.147.483.647)

CC	Carrier number	(0..3999)

0			= Scanner at induction unit

1-3999	= Carrier number

M	Mode	0	= Normal

1	= Scanner Test

S	Status	0	= Scanner response

3	= Index error

4	= No answer from scanner

5	= Scanner failure

L	Response length	Number of ASCII characters in response

(S < > 0  => L = 0).

a1a2....all-1all	Code	Response in ASCII (0..127)

File layout:

| Type |  |
| --- | --- |
| Length |  |
| U | U |
| X | X |
| G | G |
| G | G |
| C | C |
|  | M |
|  | S |
|  | L |
| a1 | a2 |
| a(len-1) | a(len) |

When the CMC has requested a Code Reader Unit (CRU) to read the code on an item, and the result of the reading is received, the code received is sent to the CSC/Host for conversion.

The result may also be retrieved from a keyboard attached to the CMC.

The response field is only present and valid if the status field is 0.

The response from the scanner or keyboard is sent directly through the CMC to the CSC/Host and can contain several barcodes. This response will contain information about good read, no read and multi labels: 0 (zero) in the length field of the barcode response indicates a no read, 99 indicates a multi-label and a value between 1 and 98 indicates a good read.

For detailed information of the length field of the scanner response, please consult the scanner protocol for the scanner used.

If CRU is mounted on induction unit and the induction unit stops on listed error, the error also will be set in the CMC status file.

If the scanner result is part of a scanner test, mode is set to 1 (Scanner Test) in the telegram.

If the scanner is placed at an induction unit, the scanner result may be sent to the CSC/Host before an ‘Item Inducted’ telegram (section ‎4.2.35) has been sent. Please note that the item may disappear or must be removed from the induction unit after the scanner result is sent and before the item reaches the sorter. If an item disappears or the induction units stops due to an error the CSC/Host is informed in a ‘Code Not Used’ telegram (see section ‎4.2.15).

If the scanner is placed at an induction unit carrier number 0 (zero) is used in the telegram.

### Scale result (27)

The scale result telegram contains a non-legally relevant part and a legally relevant part. Depending on the scale the CMC must use different protocols for transmission of the scale result, and the legally relevant part is different between the protocols, and therefore the overall length of the scale result telegram is different.

The protocols in questions are [SCALE-P30], [SCALE-P31] and [SCALE-P32]. The reader is referred to the legally relevant documentation [Alibi Messages] for the original detailed specification of these protocols.

CMC => CSC/Host

Channel:	Routing

Format [SCALE-P30]: (Legally relevant in bold)

TTLL_IXXGGGGxxspwwwwdduicc

Format [SCALE-P31] / [SCALE-P32]: (Legally relevant in bold)

TTLL_IXXGGGGxxspwwwwdduimcc

TT		27 = Telegram Type.

LL		Telegram Length	Depending on the protocol:

[SCALE-P30] length = 13 words

[SCALE-P31] / [SCALE-P32] length = 14 words

I		Induction unit number	(0..99)

XX		Index number	(0..4999)

This index is identical to xx in the weight data below.

GGGG	Item ID number	(0..2.147.483.647)

The remaining part of the telegram is a transparent copy of the weigh data received from the scale. The weight data are all in binary format.

xx		Index number	2 bytes – legal values are in the range from 0x0000 to 0xFFFF.

s		Scale status	Corresponds to DB4. Please refer to the project-specific alarm sheet provided by the scale supplier. The Scale status is 1 byte – legal values are in the range from 0x00 to 0xFF. Scale status 0x00 means no error and weight OK whereas values higher than 0x00 indicate an error (more details can be found in [SCALE-P30,P31,P32])

p		Decimal point position	1 byte:

| 0x00 | XXXXX |
| --- | --- |
| 0x01 | XXXX.X |
| 0x02 | XXX.XX |
| 0x03 | XX.XXX |
| 0x04 | Etc. |

wwww	Weight		4 bytes – legal weight values are in the range from 0x00000000 to 0xFFFFFFFF.

dd		Event counter	2 bytes – The event counter describes changes in parameters/software and calibration.  Legal byte values are in the range from 0x00 to 0xFFFF.

u		Unit of measurement	1 byte:

| 0x00 | Kilogram |
| --- | --- |
| 0x01 | Lb |
| 0x02 | Gram |
| 0x03 | Ounces |
| > 0x04 | Not defined |

i		Scale ID		1 byte – the scale ID is a unique identification of the scale at system level.

m		Weighing mode	1 byte (Note that this)

| 0x00 | Dynamic mode (AWI) |
| --- | --- |
| 0x01 | Static mode (SAWI) |

cc		CRC for weigh data	2 bytes - the CRC for weight data is a 16 bit Cyclic Redundancy Check field. The CRC is calculated by the scale and covers the fields: xxspwwwwddui.

The CRC is used to protect the weight data, so it is not possible to tamper with the weight data without it being discovered.

File layout [SCALE-P30]:

| Type |  |
| --- | --- |
| Length |  |
|  | I |
| X | X |
| G | G |
| G | G |
| X | x |
| S | p |
| w | w |
| w | w |
| d | d |
| u | i |
| c | c |

File layout [SCALE-P31] and [SCALE-P32]:

| Type |  |
| --- | --- |
| Length |  |
|  | I |
| X | X |
| G | G |
| G | G |
| x | x |
| s | p |
| w | w |
| w | w |
| d | d |
| u | i |
| c | m |
|  | c |

Note that the last byte MSB is not used.

When an induction unit (if scale equipment is installed) has finished the weighing of an item, a Scale Result (27) telegram is sent to the CSC/Host, containing the weight data exactly as they are received from the scale.

If the scale is mounted on the induction unit and the induction unit stops due to one of the scale statuses listed above, the error will be set in the CMC status file too.

The Scale Result contains a Scale status which has a general interpretation:

s	Scale status		1 byte - legal values are in the range from 0x00 to 0xFF. A status larger than 0x00 indicates an error.

The table below holds the defined scale status depending on the protocol used between CMC and scale:

| s | Scale Status using [SCALE-B020] protocol |
| --- | --- |
| 0 | OK. |
| 1 | Negative weight value. |
| 2 | Overweight item. Item is heavier than the protocol maximum weight. |
| 3 | Zero error. Scale is unable to zero. |
| 4 | No WEIGH_NOW signal detected. There is no weight value present for this index because the scale has not seen a transition of the WEIGH_NOW signal. |
| 5 | Measuring not finished. There is no weight value present for this index. |
| 6 | Weight calculation not finished. There is no weight value present for this index. |
| 7 | Scale is in calibration mode or test mode. Scale has to be in normal mode in order to weigh. |
| 8 | Item too long. |
| 9 | More than one item on the scale belt. This error is returned when an item cannot be weighed because an item has been seen in the PEC while another parcel is being weighed. This error can only be returned when the scale is configured to weigh single parcels, i.e. not allowing more than one item on the scale-belt at the same time (weighing single parcels). |
| 10 | Underweight item detected. The item is below the protocol minimum weight. |
| 11 | Reserved. |
| 12 | Calibration constant(s) or specific gravity is invalid: The scale has lost its calibration or has never been calibrated before. A defective battery in the scale can cause this. |
| 13 | Load cell error. One or more load cells are not connected correctly or are deficient. |
| 14 | Weight buffer error. The weight values needed to calculate the weight were not present in the scale's internal weight buffer. |
| 15 | Power up error. This status code signals that the scale has one or more power-up errors. Most likely the number of load cells or the presence of an accelerometer is configured wrong. Also, the power up zeroing range could have been exceeded during power up, so the scale did not zero during power-up. |
| 16 | Weight integration period too short. This error is returned when the calculated weighing time for an item is lower than the set minimum weighing time. |
| 17 | Item missing. This status code indicates that an item did not arrive at PEC 2 in the calculated time window. This status code is only returned when the scale is configured to use two PECs. |
| 18 | Too many items detected in PEC. This error is returned when an item cannot be weighed because three or more items have been seen in the PEC and all three items are still present on the weighing plate (therefore the middle item cannot be weighed). This error can only be returned when the scale is configured to allow more than one item on the scale belt at the same time (weighing multiple/several items). |
| 19 | Unknown index. The scale has been requested for data for an index not present in its buffer. |
| 20 | Warm-up delay. This error code signals that power has been applied to the scale recently and the scale is not yet ready to make correct weighing. |
| 21 | Item outside weighing belt. This error code signals that the parcel is not placed correctly on the weighing belt. |
|  | Scale Status using [SCALE] protocol |
|  | Priority 1 Errors |
| 40 | Scale is in warm-up condition. |
| 41 | Not finished with initialization (Init not received or not finished executing init). |
| 42 | Spare |
| 43 | Spare |
| 44 | Spare |
| 45 | Spare |
| 46 | Spare |
| 47 | Spare |
|  | Priority 2 Errors |
| 48 | No index telegram received before weighing. |
| 49 | Multiple indexes received before detective an item in the “Weigh_now” photocell. |
| 50 | Index already in use. |
| 51 | Index telegram error detected. (framing/parity or length/context). |
| 52 | Index buffer overflow. |
| 53 | Spare |
| 54 | Spare |
| 55 | Spare |
|  | Priority 3 Errors |
| 56 | Unable to zero, the gross weight too small. |
| 57 | Unable to zero, the gross weight too large. |
| 58 | Invalid measurement (Intern overload or underload while measured) |
| 59 | Out of weighing range |
| 60 | Spare |
| 61 | Spare |
| 62 | Spare |
| 63 | Internal scale error. (*1. See remark). |
|  | Priority 4 Errors |
| 64 | Overweight item detected. The item is above the maximum weight (range 2). |
| 65 | Overweight item detected. The item is above the maximum weight (range 1). |
| 66 | Item too long. |
| 67 | More than one item on the weighing belt. This error is returned when an item cannot be weighed because the distance to the subsequent item is too short. |
| 68 | No WEIGH_NOW signal detected. There is no weight value present for this index because the scale has not seen a transition of the WEIGH_NOW signal. The item may be too low. |
| 69 | Spare |
| 70 | Spare |
| 71 | Spare |
|  | Priority 5 Errors |
| 72 | Weight integration period too short. This error is returned when the calculated weighing time for an item is lower than the minimum weighing time set. |
| 73 | Too many items detected in PEC. This error is returned when an item cannot be weighed because three or more items have been seen in the PEC (therefore the middle item cannot be weighed). |
| 74 | Unknown item detected (stray item). This error code indicates that an item was not seen in the inlet PEC only in the outlet PEC. |
| 75 | Item missing. This error code indicates that an item did not arrive at the outlet PEC. |
| 76 | Item outside weighing belt. This error code signals that the item is not placed correctly on the weighing belt (nonautomatic weighing only). |
| 77 | Spare |
| 78 | Spare |
| 79 | Spare |
|  | Priority 6 Errors |
| 80 | Underweight item detected. The item is below the minimum weight. |
| 81 | Spare |
| 82 | Spare |
| 83 | Spare |
| 84 | Spare |
| 85 | Spare |
| 86 | Scale is in calibration/test mode. Scale has to be in normal mode in order to weigh. |
| 87 | Scale is not sealed |
| 0 | No error |
| 88-127 | Spare |
| >128 | Are not defined. |

*1: If Error code is number 63, the scale must also send telegram “Error Telegram” (see [SCALE]) with more detailed description of the error.

### Volume result (28)

Part of the Volume Result Telegram is legally relevant. The reader is referred to the legally relevant documentation [Alibi Messages] for at detailed specification. Depending on the volume scanner, the CMC must use different protocols for transmission of the volume measurement, the legally relevant part is different between the protocols, and therefore the overall length of the volume result telegram has different length.

The protocols in questions are [VOLUME-P15] and [VOLUME-P17]. The reader is referred to the legally relevant documentation [Alibi Messages] for the original detailed specification of these protocols.

CMC => CSC/Host

Channel:	Routing

Format [VOLUME-P17]: (Legally relevant in bold)

TLL_NUUXXGGGGCCxxxxssuuddvvvvvvmmbbhhhhwwwwllllttttiirrrryyyymmddhhmmsscccccc

Format [VOLUME-P15] without timestamp is used for older projects: (Legally relevant in bold)

TLL_NUUXXGGGGCCxxxxssuuddvvvvvvmmbbhhhhwwwwllllttttiirrrrcccccc

TT		28 = Telegram Type.

LL		39 = Telegram Length.

N		Volume status	0 = No error. Volume result OK

1 = Scanner/Volume measuring error (see Status later)

2 = Index error – Unknown

3 = Index error – Wrong order

4 = 	No answer from volume scanner

5 =	Scanner Failure

6 =	Checksum error in telegram received from Volume scanner

UU		Unit number	(4100-4199)	VolumeScanner number

XX		Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

CC		Carrier number	(0..3999)

0		Volume measured at induction unit

1..3999 Carrier number

The remaining part of the telegram is a transparent copy of the volume data received from the volume scanner. The volume data are all in ASCII format (0..127):

xxxx	Index number	The index number is the number received in the corresponding index telegram.

ss		Result of volume scanning (corresponds to <STATUS>.

00:	OK

01..n: Vendor-/project-specific error numbers supplied by the scanner supplier

<uu>	Unit of measurement:	00:	Volume = mm3

01:	Volume = cm3

02:	Volume = dm3

03:	Volume = m3

<dd>	Decimal Point Position for Volume result

00:  XXXXXX

01:	 XXXXX.X

02:	 XXXX.XX

03:	 XXX.XXX

04:	 XX.XXXX

05:	 X.XXXXX

06:	 0.XXXXXX

07:	 0.0XXXXXX

08:	 0.00XXXXXX

09:	 0.000XXXXXX

10:	 0.0000XXXXXX

11:	 0.00000XXXXXX

12:	 0.000000XXXXXX

13:	 0.0000000XXXXXX

14:	 0.00000000XXXXXX

15:	 0.000000000XXXXXX

<vvvvvv>	Real volume in <Units> (uu)

<mm>		Unit of measurement:	00:	H/W/L = mm

01: 	H/W/L = cm

02:		H/W/L = dm

03:		H/W/L = meter

<bb>		Decimal Point Position for Box Height/Width/Length result

00:	XXXXXX

01-15: etc. – as for <Decimal Point Position for Volume result> (dd)

<hhhh>		Height of item in <Units>

<wwww>	Width of item in <Units>

<llll>		Length of item in <Units>

<tttt>		Level as in index telegram

<ii>			Unique Volume Scanner Identification

<rrrr>		Reference Record – actual event/tamper counter value.

<yyyymmddhhmmss>

Timestamp YYYYMMDDhhmmss of the measurement.

<cccccc>	CRC for Volume Data

The CRC is used to protect the volume data for tampering. The following volume data are protected:

p17: 	xxxxssuuddvvvvvvmmbbhhhhwwwwllllttttiirrrryyyymmddhhmmss

p15: 	xxxxssuuddvvvvvvmmbbhhhhwwwwllllttttiirrrr

All data represented in small letter (i.e. xxxxssuuddvvvvvvmmbbhhhhwwwwllllttttiirrrr) are represented as ASCII values (31 Hex = 1 etc.) as the result is passed unconditionally from the Volume Scanner to the CSC/Host please consult [VOLUME-P17] for details.

File layout [VOLUME-P17]:

| Type |  |
| --- | --- |
| Length |  |
|  | N |
| U | U |
| X | X |
| G | G |
| G | G |
| C | C |
| x | X |
| x | X |
| s | S |
| u | U |
| d | D |
| v | V |
| v | V |
| v | V |
| m | M |
| b | B |
| h | H |
| h | H |
| w | W |
| w | W |
| l | L |
| l | L |
| t | T |
| t | T |
| i | I |
| r | R |
| r | R |
| y | y |
| y | y |
| m | m |
| d | d |
| h | h |
| m | m |
| s | s |
| c | c |
| c | c |
| c | c |

When an induction unit (if volume measuring equipment installed) has finished the volume measuring of an item, a Volume Result telegram is sent to the CSC/Host containing the volume result received from the volume measuring equipment.

If volume-measuring equipment is installed on the induction unit and the induction unit stops on the listed error, the error also will be set in the CMC status file.

The Volume Result contains a volume scanning result which has a general interpretation:

ss	Result of volume scanning	2 bytes - legal values are in the range from 0x0000 to 0xFFFF. A status larger than 0x00 indicates an error.

The table below holds the defined volume scanning result depending on the vendor of the scale:

| ss | General |
| --- | --- |
| 0 | OK. |
| 01..n | Error |
|  | Volume results using Sick volume scanners |
| 1 | No index telegram received within read signal |
| 2 | Multiple Indexes received within read signal |
| 3 | Index already in use |
| 4 | Index telegram error detected. (framing/parity or length/context) |
| 5 | Still measuring item when read signal was lowered |
| 6 | More than one item in scanner field – volume for first item included |
| 9 | Internal Error which prevents scanning (Reason sent in error telegram) |
| 10 | Warm-up mode – volume data may be outside tolerances |
| 11 | Item too small to be measured under the OIML restrictions (item is smaller than the tolerances of the system) |
| 12 | Item too short to be measured (related to length) |
| 13 | Item too low to be measured (related to height) |
| 14 | Item too narrow to be measured (related to width) |
| 15 | Item too long to be measured (related to length) |
| 16 | Item too high to be measured (related to height) |
| 17 | Item too wide to be measured (related to width) |
| 18 | Item outside measuring area |
| 20 | Remission too low |
| 21 | Remission too high |
| 30 | Reserved by RVSS |
| 31 | Reserved by RVSS |
| 32 | Reserved by RVSS |
| 33 | Reserved by RVSS |

### Item arrived (29) (option)

CMC => CSC/Host

Channel: Routing

Format:	TTLL_IXXGGGG

TT	29 = Telegram Type

LL	6 = Telegram Length

I	Induction unit number	(0..199)

XX	Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | I |
| X | X |
| G | G |
| G | G |

The CMC sends an Item Arrived telegram at the moment an item enters the code PEC.

If no item ID is available, the value is 0. Then the CSC/Host can send an item ID telegram (14) to give the item another Item ID number.

### Chute status (30)

CMC => CSC/Host

Channel: Routing

Format:	TTLLDDSS

TT	30 = Telegram Type

LL	4 = Telegram Length

DD	Destination number 	(0..2999)

SS	Status	0	= free

1	= chute full

2	= jam

4	= blocking from operator

8	= ProjectSpecific_3

16	= Container Missing

32	= ProjectSpecific_3

64	= Possible CTB error

128	= No24VDC

256	= disabled

512	= blocked1 from CSC/Host

1024	= blocked2 from CSC/Host

2048	= blocked3 from CSC/Host

4096	= blocked4 from CSC/Host

8192	= blocked5 from CSC/Host

File layout:

| Type |  |
| --- | --- |
| Length |  |
| D | D |
| S | S |

The status information may either be for a “chute full” or a combination of two status types, e.g. if the "chute full" and "jam" happen simultaneously, a status SS = 3 is sent.

If one of the statuses of a chute changes, the CMC sends a chute status telegram to the CSC/Host. This telegram includes the chute number and the new status of the chute specified.

After a CMC reset or a status request telegram (31), a chute status telegram for all chutes will automatically be sent to the CSC/Host.

### Chute status request (31)

CSC/Host => CMC

Channel: Routing

Format: 	TTLL

TT	31 = Telegram Type

LL	2 = Telegram Length

File layout:

Type

Length

The CMC can be requested to send a chute status telegram for all chutes regardless of the status of the chute i.e. the statuses for blocked and free chutes are returned.

After a CMC reset or a status request telegram (31), a chute status telegram for all chutes will automatically be sent to the CSC/Host.

### Block chute (32)

CSC/Host => CMC

Channel:	Routing

Format:	TTLLDDSS

TT	32 = Telegram Type

LL	4 = Telegram Length

DD	Chute	(0..2999)

SS	Status	2	= HostBlock type 1

4 	= HostBlock type 2

8	= HostBlock type 3

16	= HostBlock type 4

32	= HostBlock type 5

512	= HostUnblock type 1

1024	= HostUnblock type 2

2048	= HostUnblock type 3

4096	= HostUnblock type 4

8192	= HostUnblock type 5

File layout:

| Type |  |
| --- | --- |
| Length |  |
| D | D |
| S | S |

When receiving a Block Chute telegram, the CMC is forced to stop sortation to a specified discharge until the CSC/Host releases the forced blocking. The chute is blocked in the same way as if the chute is blocked by a photocell.

The CSC/Host have 5 different possibilities to block the chute. This may be used to distinguish between different reasons to block the chute e.g. Weight and/or CSC/Host operator. Each block must be released separately.

The blocking signal from a photocell or switch at the discharge has a higher priority than the release signal from the CSC/Host.

### Start/stop sorter (35)

CSC/Host => CMC

Channel: Routing

Format:	TTLL_R_V

TT	35 = Telegram Type

LL	4 = Telegram Length

R	Request	1	= Start

2	= Stop Now

3	= Stop When sorter is empty

4	= Stop When Induction units are empty

V	Speed	1	= Speed 1

2	= Speed 2		(Option)

3 	= Speed 3		(Option)

4	= Speed 4		(Option)

7	= Test speed

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | R |
|  | V |

This telegram is used to start or stop the sorter.

When receiving a Start command, the CMC will start the sorter with the specified speed (Speed1..Speed4). If the sorter is running at the speed specified, no actions would be taken.

In a standard sorter system only one speed will be supported.

If the sorter is already running at a speed different from the one specified in the command, the speed will be changed to the specified speed. The induction units and discharge of items will be stopped temporally while adjusting for a new sorter speed. When the new sorter speed is reached, the induction units are automatically restarted.

Speed1 to Speed4 are the different speeds of the sorter where sortation is allowed (production speed). Typically, only one production speed is allowed for a sorter.

Furthermore, the sorter may be started at test speed – this will typically be 0.5 m/s.

When receiving a ‘Stop’ Now command, the CMC will stop the sorter immediately.

When receiving a ‘Stop When Empty’ command, the CMC will stop the sorter when all carriers are empty and all induction units are stopped.

When receiving a ‘Stop When Induction Unit Empty’ command, the CMC will internally issue a Stop When Empty command to all running induction units. The sorter will stop when all induction units have stopped.

The CMC will answer with a start/stop sorter acknowledgement telegram (37).

### Start/stop induction unit (36)

CSC/Host => CMC

Channel: Routing

Format:	TTLL_R_I

TT	36 = Telegram Type

LL	4 = Telegram Length

R	Request	1	= start

2	= stop now

3	= stop when empty

I	Induction Unit No 	(0..99)

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | R |
|  | I |

When receiving a Start telegram, the induction unit will start.

When receiving a Stop Now telegram, the induction unit stop immediately.

When a Stop When Empty telegram is received, the induction unit will stop when it is empty.

The CMC will answer with a start/stop induction unit acknowledgement telegram (38).

### Start/stop sorter acknowledgement (37)

CMC => CSC/Host

Channel: Routing

Format:	TTLL_R_V_S

TT	37 = Telegram Type

LL	5 = Telegram Length

R	Request	1	= Start

2	= Stop Now

3	= Stop When Sorter is empty

4	= Stop When Induction units are empty

V	Speed	1	= Speed 1

2	= Speed 2		(Option)

3	= Speed 3		(Option)

4	= Speed 4		(Option)

7	= Test speed

S	Status	0	= OK

> 0	= error

1	= Error prevents sorter to start

2	= Illegal speed specified

3	= Sorter Blocked

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | R |
|  | V |
|  | S |

CMC sends the telegram as an acknowledgement to the start/stop sorter telegram (35). The status parameter will reflect the reason if the sorter is not able to start. The status OK means the sorter will start or stop.

### Start/stop induction unit acknowledgement (38)

CMC => CSC/Host

Channel: Routing

Format:	TTLL_R_I_S

TT	38 = Telegram Type

LL	5 = Telegram Length

R	Request	1	= start

2	= stop

3	= stop when empty

I	Induction Unit No 	(0..99)

S	Status	0	= OK

> 0	= error

1	= Error prevents induction unit to start

2	= Unknown induction unit specified

3	= Induction Unit Blocked

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | R |
|  | I |
|  | S |

CSC/Host sends the telegram as an acknowledgement to the start/stop/ induction unit telegram (36).

If the induction unit is either unknown or unable to start, the status will reflect this.

### Item inducted (40)

CMC => CSC/Host

Channel: Routing

Format:	TTLL_S_IDDXXGGGGCC_N

TT	40 = Telegram Type

LL	10 = Telegram Length

S	Status	0 =	Normal item

1 =	Stray item on induction unit

2 =	Stray item on sorter

I	Induction Unit number	(0..199)

DD	Destination number	(0..4199)

0..2999 	= Chute 0-2999

3000..3089	= Item parked on sorter

3090..3099	= CMC Item parked on sorter

4000..4098 	= Scanner number 0-98

4099 	= Any scanner

4100..4198	= Volume scanner No. 0-98

4199		= Any volume scanner

XX	Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

CC	Carrier number	(1..3999)

N	Carrier count	(1..5)

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | S |
|  | I |
| D | D |
| X | X |
| G | G |
| G | G |
| C | C |
|  | N |

The CMC sends an Item Inducted telegram as soon as the item is inducted on the sorter.

An item detected by the "stop photocell" on the induction unit without being detected by the "code photocell" is considered a "stray item", and an Item Inducted telegram (40) with status = 1 for stray item on induction unit is sent.

If an item is detected by the "manual induction unit" on the sorter and there is no code in the code queue for the manual induction unit, the item is considered a "stray item", and an Item Inducted telegram (40) with status = 1 for stray item on induction is sent.

An item detected by the "stray parcel supervision" on the sorter is considered a "stray item", and an Item Inducted telegram (40) with status = 2 for stray item on sorter is sent.

NOTE: The SPS must be in keyboard mode in order to induct items with status "Stray". If set to e.g. dispatch mode, the status for the item will be "Normal".

An item that has been detected by the "code photocell" on the separation belt but never reaches the "stop photocell" on the induction unit is considered disappeared, and a Code Not Used telegram (17) with error 1 is sent.

The CMC will assign one of the ‘CMC Item Parked on Sorter’ destinations to items which no other destination could be assigned – e.g. due to an invalid dump chute configuration.

### Item redirect (41)

CSC/Host => CMC

Channel: Routing

Format:	TTLLDDXXGGGGCC

TT	41 = Telegram Type

LL	6 = Telegram Length

DD	Destination number	(0..4099)

0..2999	 = Chute 0-2999

3000..3089	= Item parked on sorter

4000..4098 	= Scanner number 0-98

4099 	= Any scanner

4100..4198	= Volume scanner No. 0-98

4199		= Any volume scanner

XX	Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

File layout:

| Type |  |
| --- | --- |
| Length |  |
| D | D |
| X | X |
| G | G |
| G | G |

When the CSC/Host has converted a code reader telegram or wants to redirect an item, the result of the conversion/redirection must be sent to the CMC.

The CMC only uses the Index number for redirection of an item. Item ID number is not checked or used.

### Item redirect acknowledgement (42)

CMC => CSC/Host

Channel: Routing

Format:	TTLLXXGGGGCC_SDD

TT	42 = Telegram Type

LL	8 = Telegram Length

XX	Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

CC	Carrier number	(0..3999)

0			= Item at induction unit

1..3999	= Carrier number

S	Reason	0	= OK

1	= Index error

2	= Destination error

3	= Item discharged/gone

4	= Wrong side of sorter (Double tray only)

DD	Destination number	(0..4199)

0..2999 	= Chute 0-2999

3000..3089	= Item parked on sorter

3090..3099	= CMC Item parked on sorter

4000..4098 	= Scanner number 0-98

4099 	= Any scanner

4100..4198	= Volume scanner No. 0-98

4199		= Any volume scanner

File layout:

| Type |  |
| --- | --- |
| Length |  |
| X | X |
| G | G |
| G | G |
| C | C |
|  | S |
| D | D |

When the CMC has received an item redirect telegram, the contents of the telegram are checked and an Item Redirect acknowledgement telegram is sent to the CSC/Host.

If the telegram contents are accepted, an Item Redirect acknowledgement telegram with reason 0 (OK) is sent.

If the telegram contents are illegal, an Item Redirect acknowledgement telegram with a reason greater than 0 (not OK) is sent:

| Index error | The index number is invalid |
| --- | --- |
| Destination error | The destination number is not valid |
| Item discharged/gone | The item has already been sorted but not yet verified at the SPS |
| Wrong side of sorter | The destination cannot be reached from the carrier or induction unit where the item is placed. |

Carrier number 0 is used if the item is at an induction unit.

### Item discharged (43)

CMC=>CSC/Host

Channel	: Routing

Format:	TTLLDDXXGGGGCC_R

TT	43 = Telegram Type

LL	8 = Telegram Length

DD	Chute number	(0..2999)

XX	Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

CC	Carrier number	(1..3999)

R	Recirculation count	(0..255)

File layout:

| Type |  |
| --- | --- |
| Length |  |
| D | D |
| X | X |
| G | G |
| G | G |
| C | C |
|  | R |

When an item is discharged at the chute, an Item Discharged telegram is sent to the CSC/Host.

It should be noticed that the carrier is not yet supervised and confirmed as being empty.

Recirculation count informs the CSC/Host how many recirculations the item has done before the item was sorted.

### Discharge supervision (44)

CMC => CSC/Host

Channel: Routing

Format:	TTLLDDXXGGGGCC_S

TT	44 = Telegram Type

LL	8 = Telegram Length

DD	Destination number	(0..4199)

0..2999	= Chute 0-2999

3000..3089	= Item parked on sorter

4000..4098	= Scanner number 0-98

4099		= Any scanner

4100..4198	= Volume scanner No. 0-98

4199		= Any volume scanner

XX	Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

CC	Carrier number	(1..3999)

S	Status	0	= normal item supervision

1	= carrier disabled

2	= item displaced

File layout:

| Type |  |
| --- | --- |
| Length |  |
| D | D |
| X | X |
| G | G |
| G | G |
| C | C |
|  | S |

When a “discharge supervision” has checked the discharged carrier at the DS-photocell and confirmed it as being empty, a discharge supervision telegram is sent to the CSC/Host. The CSC/Host can now consider this item (index number) sorted if the status equals 0.

A disabled carrier with a not discharged item will send discharge supervision telegram with status 1 (carrier disabled). The item will not be sorted but the Index number will be released.

A displaced item will send a discharge supervision telegram with status 2 (item displaced). The item will not be sorted but the Index number will be released.

### Item sortation event (45)

CMC => CSC/Host

Channel: Routing

Format:	TTLLDDXXGGGGCC_SHHNN

TT	45 = Telegram Type

LL	10 = Telegram Length

DD	Destination number	(0..4199)

0..2999 	= Chute 0-2999

3000..3089	= Item parked on sorter

3090..3099	= CMC Item parked on sorter

4000..4098 	= Scanner number 0-98

4099 	= Any scanner

4100..4198	= Volume scanner No. 0-98

4199		= Any volume scanner

XX	Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

CC	Carrier number	(1..3999)

S	Reason	0	= Chute blocked

1	= Max. Recirculation

2	= No Activation/Possible CTB Error

3	= Not code-converted

4	= No answer from scanner

5	= Item not sorted due to tray spacing

6	= Item on Activated Carrier

7	= RNI Not Present

8	= Destination Invalid for Multi Carrier item

9	= One or more Multi-Dest invalid

10	= Vertical tray spacing

11	= Redirected by CMC

HH	Chute Status	0	= free

1	= chute full

2	= jam

4	= blocking from operator

8	= ProjectSpecific_1

16	= ProjectSpecific_2

32	= ProjectSpecific_3

64	= Possible CTB error

128	= No24VDC

256	= disabled

512	= blocked1 from CSC/Host

1024	= blocked2 from CSC/Host

2048	= blocked3 from CSC/Host

4096	= blocked4 from CSC/Host

8192	= blocked5 from CSC/Host

NN		New Destination	(0..4099)

0..2999 	= Chute 0-2999

3000..3089	= Item parked on sorter

3090..3099	= CMC Item parked on sorter

4000..4098 	= Scanner number 0-98

4099 	= Any scanner

4100..4198	= Volume scanner No. 0-98

4199		= Any volume scanner

File layout:

| Type |  |
| --- | --- |
| Length |  |
| D | D |
| X | X |
| G | G |
| G | G |
| C | C |
|  | S |
| H | H |
| N | N |

If, for some reason, an item is acting abnormally while sorting to its destination, an item sortation event telegram is sent to the CSC/Host. The telegram includes a reason for the event.

The CSC/Host can then redirect the item to another destination.

Each time the item is recirculated, the number of recirculations is incremented by one in CMC.

The point where the CMC is incrementing the counter is the physical location of the IU/SPS from which the item was originally inducted.

The reason field contains the following information:

| Reason | Explanation | Result |
| --- | --- | --- |
| Chute Blocked | Chute is full, jammed or blocked by other reasons | Item will be recirculated |
| Max.Recirculation | Item has recirculated Max.Reciculation times | Item will be redirected to the Max.Recirc. dump chute |
| No Activation/ Possible CTB Error | An item has been found on a carrier, which has been discharged. The carrier reports that no activation has been performed. | Item will be recirculated |
| Not Code Converted | If the CMC has not received a destination of a scanned item at a certain point the CSC/Host is informed. | Item will be recirculated and rescanned |
| No Answer from Scanner | No answer has been received from scanner for an item | Item will be recirculated and rescanned |
| Item not sorted due to tray spacing | The item could not be sorted due the chute was blocked due to tray spacing. | Item will be recirculated |
| Item on Activated Carrier | Verification of an activated carrier at an SPS has found an item on the carrier. Carrier reports that it has been activated. | Item will be recirculated |
| RNI Not Present | The Ready Next Item (RNI) signal from a conveyor line is not present at the time an item must be sorted to the destination. This is only used when spacing and/or tracking of sorted item must be performed. | Item will be recirculated |
| Invalid destination for multi carrier item | If a multi-carrier item has the destination for a single carrier chute this event will be sent to the CSC. | Item will be redirected to the Max.recirc dump chute |
| One or more Multi-Dest are invalid | When receiving the ‘Item Redir multi dest’ telegram the CMC will check the destinations and if any illegal destinations are found in the telegram, this event is forwarded to the CSC/Host. | The CMC will only use the valid destinations. |

If the CMC redirects the item, the new destination will be inserted in the new destination field. If the item is not redirected, the destination and new destination will be the same.

The chute status information may be a combination, e.g. if the "chute full" and "jam" happen simultaneously, a status HH = 3 is sent. If the destination is not a chute, the chute status information is set to 0.

The CMC will assign one of the ‘CMC Item Parked on Sorter’ destinations to items which no other destination could be assigned – e.g. due to an invalid dump chute configuration.

Reason 8 (Destination Invalid for Multi Carrier item) and reason 9 (One or more Multi-Dest invalid) are used when multi-carrier items are directed to chute destinations only capable of receiving single carrier items. The reasons may be sent when the item is inducted on the sorter or when it is being redirected.

### Item disappeared (46) (option)

CMC => CSC/Host

Channel: Routing

Format:	TTLL_IXXGGGGCC

TT	46 = Telegram Type

LL	7 = Telegram Length

I	Stray parcel supervision		(100..199)

XX	Index number		(0..4999)

GGGG	Item ID number		(0..2.147.483.647)

CC	Carrier number		(1..3999)

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | I |
| X | X |
| G | G |
| G | G |
| C | C |

When the CMC, via a stray item supervisor, detects an item missing on the sorter, an item disappeared telegram is sent to the CSC/Host. The CSC/Host will consider the received index number as having been released.

### Item redirect-multi dest (47)

CSC/Host => CMC

Channel: Routing

Format:	TTLLXXGGGG_SPP_NAABBCCDDEE

TT	47 = Telegram Type

LL	13 = Telegram Length

XX	Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

S	Mode	0	= Normal

1	= Nearest Destination

PP	Destination number	(0..4199)

0..2999	 = Chute 0-2999

3000..3089	= Item parked on sorter

4000..4098 	= Scanner number 0-98

4099 	= Any scanner

4100..4198	= Volume scanner No. 0-98

4199		= Any Volume scanner

N	No. of Alt. Destinations		(0..5)

AA	Alternative Destination 1		(0..2999) = Chute 0-2999

BB	Alternative Destination 2		(0..2999) = Chute 0-2999

CC	Alternative Destination 3		(0..2999) = Chute 0-2999

DD	Alternative Destination 4		(0..2999) = Chute 0-2999

EE	Alternative Destination 5		(0..2999) = Chute 0-2999

File layout:

| Type |  |
| --- | --- |
| Length |  |
| X | X |
| G | G |
| G | G |
|  | S |
| P | P |
|  | N |
| A | A |
| B | B |
| C | C |
| D | D |
| E | E |

When the CSC/Host has converted a code reader telegram or want to redirect an item, the result of the conversion/redirection must be sent to the CMC.

The CMC only uses the Index number for redirection of an item. The item ID number is not checked or used.

The ‘Redirect Item Multi-Dest’ telegram will be acknowledged with an ‘Item Redirect Acknowledge’ telegram if all parameters are accepted by the CMC. Otherwise the reason will be reflected in the status of the acknowledge telegram.

Only destinations of the same type must be used in the telegram – i.e. if a scanner and a destination are included in the telegram, the ‘Item Redirect Multi-Dest’ telegram is rejected.

Only chute destinations are allowed in the alternative destinations 1-5.

The field ‘No. of alternative destinations’ specifies how many valid alternative destinations (up to 5) are included in the telegram – i.e. if the number 2 is specified the alternative destination 3, 4 and 5 is not used or checked by the CMC.

The destination used in all other telegrams will always be the current destination of the item.

The ‘Mode’ field has the following meaning:

| Normal: | The Destination in ‘PP’ will be used as the first destination of the telegram. If the item cannot be sorted to this destination, the first alternative destination (specified in the field ‘AA’) will be the next destination for the item and so on. When the CMC has tried the alternative destination 5 without success, the original destination will be used again. The CMC will try to sort the item switching between the destinations in turn until the item has recirculated ‘Max. recirculation’ times – after this the item is sorted to the Max. Recirculation dump chute. |
| --- | --- |
| Nearest Destination: | The CMC will evaluate which of the specified destination is nearest at the time this telegram is received and direct the item to this destination. If the item cannot be sorted to this destination, the CMC will use the next destination specified in the telegram – e.g. if the ‘Alternative destination 2’ was used, the next destination will be ‘Alternative destination 3’. When the mode ‘Nearest Destination is used, the destinations must be listed in numerical order – i.e. the numerical smallest destination number in the field ‘Destination’ (‘DD’) and the numerical highest number in the field ‘Alternative Destination 5’ (‘EE’). |

### Carrier passed SPS (48)

CMC => CSC/Host

Channel: Routing

Format:	TTLL_S_IDDXXGGGGCC_RNN_T

TT	48 = Telegram Type

LL	12 = Telegram Length

S	SPS number	(100-199)

I	Induction Unit number	(0..199)

DD	Destination number	(0..4199)

0..2999	= Chute 0-2999

3000..3089	= Item parked on sorter

3090..3099	= CMC Item parked on sorter

4000..4098 	= Scanner number 0-98

4099 	= Any scanner

4100..4198	= Volume scanner No. 0-98

4199		= Any volume scanner

XX	Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

CC	Carrier number	(1..3999)

R	Recirc count	(0..199)

NN	Carrier count	(0..3)

T	Carrier status	(0..3)

0	= Free

1	= Occupied

2	= Fault disabled

3	= Discharged

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | S |
|  | I |
| D | D |
| X | X |
| G | G |
| G | G |
| C | C |
|  | R |
| N | N |
|  | T |

The CMC sends a carrier passed SPS telegram at the moment the carrier passes an SPS and the log (CarrierPassedSPS) for SPS is selected. This telegram may be deactivated by deactivating the log (CarrierPassedSPS).

### Carrier data reboot (60)

CSC/Host => CMC

Channel: Routing

Format:	TTLLCC_IDDXXGGGG_A_B_C_D_N

TT	60 = Telegram Type

LL	13 = Telegram Length

CC	Carrier number	(1-3999)

I	Induction Unit number	(0..199)

DD	Destination	(0..4199)

XX	Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

A	Moved Out	(0..1)

B	Code converted	(0..1)

C	Disabled.	(0..1)

D	Not used	(0..1)

N	Carrier Count	(1..5)

File layout:

| Type |  |
| --- | --- |
| Length |  |
| C | C |
|  | I |
| D | D |
| X | X |
| G | G |
| G | G |
|  | A |
|  | B |
|  | C |
|  | D |
|  | N |

A Carrier Data Telegram is sent for each occupied carrier after a Hard Reset telegram (0) to update the carrier data table in CMC if the CMC requests it as part of the initialisation. The Request Carrier Data Reboot telegram will be sent from the CMC in this case.

If the field ‘Moved Out’ is set to 1 (one), it indicates that the item has been discharged (sorted) but the following Stray Parcel Detector has not yet supervised the carrier. If the ‘Moved Out’ field is 0 (zero), it indicates that the item has been inducted but not yet sorted.

### Carrier data reboot acknowledgement (61)

CMC => CSC/Host

Channel: Routing

Format:	TTLLCCXXGGGG_S

TT	61 = Telegram Type

LL	7 = Telegram Length

CC	Carrier number	(1..3999)

XX	Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

S	Reason	0	= OK

1	= Index error

2	= Destination error

3	= Carrier unknown

4	= Carrier Table Error

5	= Wrong sorter side (Double tray only)

File layout:

| Type |  |
| --- | --- |
| Length |  |
| C | C |
| X | X |
| G | G |
| G | G |
|  | S |

When the CMC has received a carrier data telegram, the contents of the telegram are checked and a carrier data acknowledgement telegram is sent to the CSC/Host.

If the telegram contents are accepted, a carrier data acknowledgement telegram with reason 0 (OK) is sent.

If the telegram contents are illegal, a carrier data acknowledgement telegram with a reason greater than 0 (not OK) is sent.

### Request carrier data reboot (63)

CMC => CSC/Host

Channel: Routing

Format:	TTLL

TT	63 = Telegram Type

LL	2 = Telegram Length

File layout:

Type

Length

After a hard reset, the CMC may request the carrier data from on the Routing channel. The CSC/Host or external host will then download carrier data in a number of Carrier Data Reboot telegrams.

If no system holds the Carrier Data, this telegram will not be sent.

### Carrier data transmitted (64)

CSC/Host => CMC

Channel: Routing

Format:	TTLL

TT	64 = Telegram Type

LL	2 = Telegram Length

File layout:

Type

Length

The carrier data table (reboot) is sent to the CMC before Carrier data Transmitted Telegram (64) is sent. CMC can start.

CMC will answer with ready telegram (05).

See section ‎5.2.1 for a complete scenario of the CMC initialization sequence.

### Index status (65)

CMC => CSC/Host

Channel: Routing

Format:	TTLL_PCC_IDDXXGGGG_A_B_C_D_N

TT	65 = Telegram Type

LL	14 = Telegram Length

P	Location	0	= Sorter

1	= Induction Unit

CC	Carrier number	(0-3999)

I	Induction unit number	(0..199)

DD	Destination	(0..4199)

XX	Index number	(0..4999)

GGGG	Item ID number	(0..2.147.483.647)

A	Moved Out	(0..1)

B	Not used	(0..1)

C	Not used	(0..1)

D	Not used	(0..1)

N	Carrier Count	(0..5)

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | P |
| C | C |
|  | I |
| D | D |
| X | X |
| G | G |
| G | G |
|  | A |
|  | B |
|  | C |
|  | D |
|  | N |

An Index Status Telegram is sent for each active (used) index when the CSC/Host requests it through a Request Index Status telegram (66).

The Index Status telegram contains information about the item which the index represents.

If the item is located on the induction unit, the carrier number and carrier count will contain the value 0.

If the field ‘Moved Out’ is set to 1 (one), it indicates that the item has been discharged (sorted) but the following Stray Parcel Detector has yet not supervised the carrier. If the ‘Moved Out’ field is set to 0 (zero), it indicates that the item may be on the induction unit or the item has been inducted but not sorted yet.

The Location parameter will indicate where the item currently is located.

Please note: This telegram is intended to be used during initialisation of the CMC/CSC where the sorter is standing still. If the telegram is used while the sorter is running and doing sortation, inconsistencies may occur.

### Request index status (66)

CSC/Host => CMC

Channel: Routing

Format:	TTLL

TT	66 = Telegram Type

LL	2 = Telegram Length

File layout:

Type

Length

If the CSC/Host needs to be synchronized after a CSC/Host switchover or restart, it will request all indexes inclusive item data from the CMC.

The CMC will answer with an Index Status telegram (65) for each active index and an Index Data Transmitted telegram (67). If no indexes are active, only an ‘Index Data Transmitted’ telegram (67) is sent.

### Index data transmitted (67)

CMC => CSC/Host

Channel: Routing

Format:	TTLL

TT	67 = Telegram Type

LL	2 = Telegram Length

File layout:

Type

Length

This telegram is sent by the CMC when an ‘Index Status’ telegram (65) has been sent for each active index.

### Input changed (70)	(option)

CMC => CSC/Host

Channel:	Routing

Format:	TTLLPP_S

TT	70 = Telegram Type

LL	4 = Telegram Length

PP	Input number	(0..4095)

S	Input status	0	= Input Low/Off

1	= Input High/On

File layout:

| Type |  |
| --- | --- |
| Length |  |
| P | P |
|  | S |

When an input change is detected, the CMC sends a telegram to the CSC/Host if an input change request for the input is issued.

See controls specification for software and hardware for the specification of inputs and input numbers.

After a CMC reset input table, changed telegrams are sent for inputs with high signal.

### Set function (73)

CSC/Host => CMC

Channel: Routing

Format:	TTLL_TIIFF

TT	73 = Telegram Type

LL	5 = Telegram Length

T	Object Type	0	= Sorter

1	= Induction Units

2	= Chutes

3	= Carrier

4	= SPS/MAI

5	= Scanner

6	= Safety Supervision

II	Index number of object

FF	Function number	0	= Short container

1	= Medium container

2	= Tall container

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | T |
| I | I |
| F | F |

The CSC/Host can request the CMC to perform a number of functions – e.g. start an emptying sequence of a chute or inform the CMC that a certain action is allowed the next time a pushbutton is pressed.

The Function number and interval are project specific. For this project 0, 1, and 2 numbers are commands to inform the CMC which container type the operator has selected from the ATOP "up", "down" push buttons. Based on this command, CMC will use the appropriate full PEC signal out of the three installed for each direct to container chutes. This field is not used for the other chute types.

### Function status (74)

CMC => CSC/Host

Channel: Routing

Format:	TTLL_TIIFFSS

TT	74 = Telegram Type

LL	6 = Telegram Length

T	Object Type.	0	= Sorter

1	= Induction Units

2	= Chutes

3	= Carrier

4	= SPS/MAI

5	= Scanner

6	= Safety Supervision

II	Index number of object

FF	Function number	0	= Short container

1	= Medium container

2	= Tall container

SS	Function Status	(N/A)

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | T |
| I | I |
| F | F |
| S | S |

This telegram is used to inform the CSC/Host of the progress and status of a function started with the Set Function telegram (73).

The Function number and interval are project specific. For this project 0, 1, and 2 numbers are statuses to inform the CSC which container type the CMC has currently set for the direct to container chutes. Based on this function, the CSC will display on the ATOP letting know the operator which container type is currently used. This field is not used for the other chute types.

The Function status is also project specific. For this project this field is not applicable

### Set output (75) (option)

CSC/Host => CMC

Channel: Routing

Format:	TTLLPP_STT

TT	75 = Telegram Type

LL	5 = Telegram Length

PP	Output number	(0..4095)

S	Output status	0	= turn off

1	= turn on

2	= flashes slow

3	= flashes fast

4	= Cycle			(Option)

TT	Timer for cycle [10ms]	(0..9999)			(Option)

File layout:

| Type |  |
| --- | --- |
| Length |  |
| P | P |
|  | S |
| T | T |

The CSC/Host can require that the CMC sets a specified output to off, on, or flashing slow/fast.

The cycle status will set an output high at a time specified in the telegram (one-shot).

See controls specification for software and hardware for the specification of outputs and output number.

### Request max. recirculation (90)

CSC/Host => CMC

Channel: Routing

Format:	TTLL

TT	90 = Telegram Type

LL	2 = Telegram Length

File layout:

Type

Length

The CSC/Host may request the Max. Recirculation counter from the CMC. The CMC will return a Max. Recirculation telegram (91).

### Max. recirculation (91)

CSC/Host => CMC

CMC => CSC/Host

Channel:	Routing

Format:	TTLL_N

TT	91 = Telegram Type

LL	3 = Telegram Length

N	Max. recirculations	(0..255)

0 - 127	= Number of recirculations

>= 128	= Infinite number of recirc.

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | N |

This telegram is used to send the Max Recirculation number to the CMC, or it is received from the CMC in case the CMC has received a Request Max. Recirculation (90) telegram.

If an item fails to go to a destination due to a blocked destination, the CMC informs the CSC/Host with the Item Sortation Event telegram (45) and the CSC/Host will then redirect the item to an alternative destination. Each time the item passes the IU/SPS from which the item originally was inducted, the number of recirculations is incremented by one.

If the max. number of recirculation’s is reached, the CMC informs the CSC/Host with the Item Sortation Event telegram (45) and the CSC/Host then will redirect the item to an alternative destination.

If the CSC/Host does not redirect the item, then the item will be dumped in the Max. recirculation dump chute by the CMC (only if CMC is in Host mode).

### Dump chutes (95)

CSC/Host => CMC

CMC => CSC/Host

Channel: Routing

Format:	TTLLIIRRSSPP

TT	95 = Telegram Type

LL	Telegram Length = 6

II	Induction Unit Number	(0..199)

RR	Max.Recirculation	(0..4199)

0..2999	= Chute 0-2999

3000..3089	= Item parked on sorter

4000..4098 	= Scanner number 0-98

4099 	= Any scanner

4100..4198	= Volume scanner No. 0-98

4199		= Any volume scanner

SS	Stray Items	(0..4199)

0..2999	= Chute 0-2999

3000..3089	= Item parked on sorter

4000..4098	= Scanner number 0-98

4099		= Any scanner

4100..4198	= Volume scanner No. 0-98

4199		= Any volume scanner

PP	Scanner Problems	(0..4199)

0..2999	= Chute 0-2999

3000..3089	= Item parked on sorter

4000..4098	= Scanner number 0-98

4099		= Any scanner

4100..4198	= Volume scanner No. 0-98

4199		= Any volume scanner

File layout:

| Type |  |
| --- | --- |
| Length |  |
| I | I |
| R | R |
| S | S |
| P | P |

The CMC is able to accept the above specified dump destinations for each induction unit and SPS/MAIs.

The dump chutes are specified per induction unit – meaning that the item to dump will be directed to the dump chute for the induction unit where the item originally was inducted to the sorter. The destination may be a chute; an overhead scanner or the item may be parked on sorter.

Max.Recirculation dump destination will be used for all items that have recirculated Max.Recirculation times on the sorter.

Stray Item dump destination will be used for all stray items inducted from SPS/MAI or an induction unit. The handling of stray items on an induction unit will normally be to stop the stray item at the induction unit.

The Scanner Problem dump destination will be used if the scanner was not able to read the barcode of an item or if the CSC/Host does not redirect an item as a result of a conversion of a barcode to a destination before the carrier with the item reaches a predefined spot on the sorter after the scanner.

The dump chute destination will be used as soon they are received.

The CSC/Host may also request the dump chutes by sending a Request Dump Chute telegram (96). The CMC will then send the Dump Chute telegram to the CSC/Host.

### Request dump chutes (96)

CSC/Host => CMC

Channel: Routing

Format:	TTLL_RII

TT	96 = Telegram Type

LL	4 = Telegram Length

R	Request	0	= Upload for single induction unit

1	= Upload for all induction units

II	Induction Unit Number	(0..199)

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | R |
| I | I |

The CSC/Host may request the dump chutes from the CMC. The CMC will return a Dump Chute telegram (95) with the dump chutes for the requested sorter object.

If all dump chutes for all induction units are requested, a Dump Chute telegram for each installed induction unit and SPS/MAI will be sent.

The value of Induction Unit Number (II) is ignored in case upload for all induction units is requested.

### Set status (97)

CSC/Host => CMC

Channel: Routing

Format:	TTLLPP_SDD

TT	97 = Telegram Type

LL	5 = Telegram Length

PP	Status point	(0..127)

S	Status	0	= Off

1	= On

DD	Parameter	(0.. 32.767 = 0..7FFFH)

File layout:

| Type |  |
| --- | --- |
| Length |  |
| P | P |
|  | S |
| D | D |

The CSC/Host may set a status point as either on or off in the status data block of the CMC to have the status point displayed in an external SCADA system where only the CMC has an interface. An example of a CSC/Host status point is ‘Hard disk almost full’. The parameter may be used to transfer additional information such as how much the disk is filled, e.g. 80 (%).

During reset, the CSC/Host point is reset and it is the CSC/Host responsibility to set active points on again after the reset.

### Status (99) (option)

CMC => CSC/Host

Channel: Routing

Format :	TTLL_T_SIIDD..DD

TT	99 = Telegram Type

LL	Telegram Length

T	Object Type.	0	= Sorter

1	= Induction units

2	= CMC

S	Object Subtype

II	Index number of object

DD	1 – 96 words of data

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | T |
|  | S |
| I | I |
| D | D |
|  | ... |
| D | D |

The CMC is able to send status for objects defined in Object Type. The telegram contains an identification of the object (type, subtype and index)

The status for the devices listed below will always be sent when the status changes. The status for sorter and induction units will also be sent as response for start and stop.

The CMC can be set up to send status for all components during start up if needed.

Currently the following status is sent to the CSC/Host:

Sorter Status	Type:	0

Subtype:	0

Index:	0

Status:	Bit 0		: Stopped

Bit 1		: Starting

Bit 2		: Running

Bit 3		: Stopping

Bit 4		: SortationAllowed

Bit 5..7	: Not Used

Bit 8		: Disabled	(Option)

Bit 9		: Blocked 	(Option)

Bit 10	: Test

Bit 11	: DischargeTest

Bit 12	: LampTest

Bit 13	: CarrierToServicePlatform

Bit 14	: Not Synchron

Bit 15	: Error

Induction Unit Status	Type:	1

Subtype:	0

Index:	Induction unit number (0..99)

Status: 	Bit 0		: Stopped

Bit 1		: Starting

Bit 2		: Running

Bit 3		: Stopping

Bit 4..7	: Not Used

Bit 8		: Disabled	(Option)

Bit 9		: Blocked 	(Option)

Bit 10	: Test

Bit 11..14	: Not Used

Bit 15	: Error

CMC Status	Type:	2

Subtype:	0

Index:	CMC number  = 0

Status: 	Bit 0..7	: Not used

Bit 8	: 0	= Routing RX channel disconnected

: 1	= Routing RX channel connected

Bit 9	: 0 = Routing TX channel disconnected

: 1 = Routing TX channel connected

Bit 10..15	: Not used

### Reject telegram (100)

CMC => CSC/Host

Channel: Routing

Format:	TTLL_RNNSS

TT	100 = Telegram Type

LL	5 = Telegram Length

R	Reason	0	= Unknown telegram number

1	= Illegal length of telegram

NN	Telegram number of received telegram	(0.. 0xFFFF)

SS	Telegram length from the received telegram	(0.. 0xFFFF)

File layout:

| Type |  |
| --- | --- |
| Length |  |
|  | R |
| N | N |
| S | S |

This telegram is used to reject the reception of a telegram received at the Routing channel and is used for debugging purposes.

For a number of telegrams, no acknowledge is defined for the telegram in question. In case unknown telegrams are received in the telegram, this telegram may be used to inform the CSC/Host that the telegram sent is rejected and no action is performed by the CMC.

The telegram includes the telegram number/type and the length of the rejected telegram.

# Telegram Layout and Scenarios

This section describes a number of telegram scenarios between the CMC and the CSC/Host. The scenarios are split in two – one for normal operation and one for test operation. Not all parameters are shown for the telegrams – only the most important ones are included to ease the understanding.

## Telegram Layout

### Telegram layout – Protocol version 3

Note: The examples below are for a Siemens S7 PLC. For an Allen-Bradley PLC, be aware that when data type is a double word (item-id, sequence number etc), the word order is switched compared to the examples below.

File layout:

| Offset Total 0 | Channel Header | Channel ID = 02 |  |  |
| --- | --- | --- | --- | --- |
| 1 |  | Protocol Version = 03 |  |  |
| 2 | Telegram Header | Offset 0 | Telegram Type = 45 |  |
| 3 |  | 1 | Telegram Length = 10 |  |
| 4 | Data | 2 | D = 01H | D = 00H |
| 5 |  | 3 | X = 00H | X = 10H |
| 6 |  | 4 | G = 00H | G = 00H |
| 7 |  | 5 | G = 32H | G = 10H |
| 8 |  | 6 | C = 00H | C = 02H |
| 9 |  | 7 |  | S = 01H |
| 10 |  | 8 | H = 00H | H = 00H |
| 11 |  | 9 | N = 01H | N = 00H |

This example shows the data layout of an Item Sortation Event telegram (45) including the channel header.

The length of the telegram is including the telegram header but excluding the channel header.

The data in the Item Sortation Event are as follows:

Destination (DD) 	= 0100H	= 256D

Index (XX)		= 0010H	= 16D

Item ID (GGGG)	= 00003210H	= 12.816D

Carrier (CC)		= 0002H	= 2D

Reason (S)		= 01H	= 1D

Chute Status (HH)	= 0000H 	= 0D

New Destination (NN)	= 0100H	= 256D

The next example shows the data layout of a telegram block with two telegrams included.

File layout:

| Offset Total 0 | Channel Header | Channel ID = 02 |  |  |
| --- | --- | --- | --- | --- |
| 1 |  | Protocol Version = 03 |  |  |
| 2 | Telegram Header 1 | Offset 0 | Telegram Type = 45 |  |
| 3 |  | 1 | Telegram Length = 10 |  |
| 4 | Data 1 | 2 | D = 01H | D = 00H |
| 5 |  | 3 | X = 00H | X = 10H |
| 6 |  | 4 | G = 00H | G = 00H |
| 7 |  | 5 | G = 32H | G = 10H |
| 8 |  | 6 | C = 00H | C = 02H |
| 9 |  | 7 |  | S = 01H |
| 10 |  | 8 | H = 00H | H = 00H |
| 11 |  | 9 | N = 01H | N = 00H |
| 12 | Telegram Header 2 | Offset 0 | Telegram Type = 45 |  |
| 13 |  | 1 | Telegram Length = 10 |  |
| 14 | Data 2 | 2 | D = 01H | D = 00H |
| 15 |  | 3 | X = 00H | X = 01H |
| 16 |  | 4 | G = 00H | G = 00H |
| 17 |  | 5 | G = 32H | G = 11H |
| 18 |  | 6 | C = 00H | C = 02H |
| 19 |  | 7 |  | S = 01H |
| 20 |  | 8 | H = 00H | H = 00H |
| 21 |  | 9 | N = 01H | N = 00H |

This example shows the data layout of two Item Sortation Event telegrams (45) including the channel header sent in one telegram block.

### Telegram layout – Protocol version 4

File layout:

| Offset Total 0 | Channel Header | Channel ID = 02 |  |  |
| --- | --- | --- | --- | --- |
| 1 |  | Protocol Version = 04 |  |  |
| 2 |  | Sequence number (lower word) |  |  |
| 3 |  | Sequence number (upper word) |  |  |
| 4 | Telegram Header | Offset 0 | Telegram Type = 45 |  |
| 5 |  | 1 | Telegram Length = 10 |  |
| 6 | Data | 2 | D = 01H | D = 00H |
| 7 |  | 3 | X = 00H | X = 10H |
| 8 |  | 4 | G = 32H | G = 10H |
| 9 |  | 5 | G = 00H | G = 00H |
| 10 |  | 6 | C = 00H | C = 02H |
| 11 |  | 7 |  | S = 01H |
| 12 |  | 8 | H = 00H | H = 00H |
| 13 |  | 9 | N = 01H | N = 00H |

This example shows the data layout of an Item Sortation Event telegram (45) including the channel header.

The length of the telegram includes the telegram header but excludes the channel header.

The data in the Item Sortation Event are as follows:

Destination (DD) 	= 0100H	= 256D

Index (XX)		= 0010H	= 16D

Item ID (GGGG)	= 00003210H	= 12.816D

Carrier (CC)		= 0002H	= 2D

Reason (S)		= 01H	= 1D

Chute Status (HH)	= 0000H 	= 0D

New Destination (NN)	= 0100H	= 256D

The next example shows the data layout of a telegram block with two telegrams included.

File layout:

| Offset Total 0 | Channel Header | Channel Id = 02 |  |  |
| --- | --- | --- | --- | --- |
| 1 |  | Protocol Version = 04 |  |  |
| 2 |  | Sequence number (lower word) |  |  |
| 3 |  | Sequence number (upper word) |  |  |
| 4 | Telegram Header 1 | Offset 0 | Telegram Type = 45 |  |
| 5 |  | 1 | Telegram Length = 10 |  |
| 6 | Data 1 | 2 | D = 01H | D = 00H |
| 7 |  | 3 | X = 00H | X = 10H |
| 8 |  | 4 | G = 32H | G = 10H |
| 9 |  | 5 | G = 00H | G = 00H |
| 10 |  | 6 | C = 00H | C = 02H |
| 11 |  | 7 |  | S = 01H |
| 12 |  | 8 | H = 00H | H = 00H |
| 13 |  | 9 | N = 01H | N = 00H |
| 14 | Telegram Header 2 | Offset 0 | Telegram Type = 45 |  |
| 15 |  | 1 | Telegram Length = 10 |  |
| 16 | Data 2 | 2 | D = 01H | D = 00H |
| 17 |  | 3 | X = 00H | X = 01H |
| 18 |  | 4 | G = 32H | G = 11H |
| 19 |  | 5 | G = 00H | G = 00H |
| 20 |  | 6 | C = 00H | C = 02H |
| 21 |  | 7 |  | S = 01H |
| 22 |  | 8 | H = 00H | H = 00H |
| 23 |  | 9 | N = 01H | N = 00H |

This example shows the data layout of 2 Item Sortation Event telegrams (45) inclusive the channel header sent in one telegram block.

## Telegram Scenarios for Normal Operation

### Reset/synchronisation scenarios

The following scenario shows the reset/synchronization scenario between the CSC/Host and the CMC, which must be performed every time the connection is established between the CSC/Host and the CMC.

The following scenarios are outlined:

- Start up of CSC/Host – carrier data are downloaded (Hard reset)

- Start up of CSC/Host – No carrier data are downloaded (Soft reset)

- Start up of CMC (Hard reset)

The number of telegrams exchanged during a reset sequence depends on whether carrier data must be downloaded from the CSC/Host. Whether carrier data must be downloaded or not is a project specific option.

In the normal CSC-CMC setup carrier data must be downloaded.

#### CSC/Host start-up – carrier data are downloaded

This scenario shows the hard reset/synchronization when the CSC/Host comes up and establishes a Routing connection to an active CMC.

The following scenario is typical or CSC mode for the CMC:

(1)	The connection is established, and a reset request is sent to the CMC.

(2)	The CMC performs the initialisation and replies with a Reset Completed telegram.

(3)	The CSC/Host downloads the Max.Recirculation counter to the CMC

(4)	The CSC/Host downloads the Dump chutes to the CMC. The reason this telegram is a dotted line is that if the CSC is in control, the CMC does not redirect items at all and therefore it does not need the dump chutes.

(5)	The CSC/Host sends a Ready telegram to inform the CMC that the CSC/Host is ready to download data and receive sorting information.

(6)	The CMC request carrier data from the CSC/Host – as mentioned this is only done if the CSC/Host is the master of the carrier data and that a hard reset is performed.

(7)	The CSC/Host sends carrier data for all occupied carriers to the CMC.

(8)	When no more carrier data must be sent, a Carrier Data Transmitted telegram is sent from the CSC/Host to the CMC.

(9)	The CMC sends a Ready telegram to inform the CSC/Host that the CMC is ready to start the sorter and sortation.

(10)	The CMC informs the CSC/Host of the status of all chutes.

#### CSC/Host start-up – no carrier data are downloaded

This scenario shows the soft reset/synchronization when the CSC/Host comes up and establishes a Routing connection to an active CMC. No carrier data are transmitted. The scenario for a hard reset is the same as this soft reset scenario except there is no reason to upload index status as the carrier tables in the CMC are empty after a hard reset.

The following scenario is typical for Host mode:

(1)	The connection is established, and a reset request is sent to the CMC.

(2)	The CMC performs the initialisation and replies with a Reset Completed telegram.

(3)	Immediately after the Reset Completed telegram, the Ready telegram is sent from the CMC to the CSC/Host.

(4)	If needed the CSC/Host downloads the Max.Recirculation counter to the CMC – otherwise the predefined value in the CMC will be used.

(5)	The CSC/Host may download the Dump chutes to the CMC - otherwise the predefined dump chute values in the CMC will be used.

(6)	If the CSC/Host wants to synchronise the CSC/Host internal carrier tables with the carrier tables in the CMC, a Request Index Status telegram is sent to the CMC.

(7)	A number of index status telegrams are sent from the CMC to the CSC/Host – one telegram is sent for each active index.

(8)	When all ‘Index Status’ telegrams have been sent, an Index Data Transmitted telegram is sent from the CMC. (6) – (8) may be skipped if no synchronisation needs to take place.

(9)	The CSC/Host sends a Ready telegram to inform the CMC that the CSC/Host is ready to receive sorting information.

(10)	The CMC informs the CSC/Host of the status of all chutes.

#### CMC start-up

If the PLC of the CMC is restarted the Reset Completed telegram (2) will be send without the CSC/Host has asked for a reset. The reset type performed may be either Hard or soft depending on the state of the CMC. The following scenario is typical for CSC mode:

(1)	The connection will come up at an early point. The CSC/Host may send a Reset at this point, but it will be ignored by the CMC

(2)	When the CMC has finished its internal initialisation, a Reset Completed telegram is sent. Please note that even if the CSC/Host has asked for a soft reset, the CMC will perform a hard reset.

(3)	The CSC/Host downloads the Max.Recirculation counter to the CMC

(4)	The CSC/Host downloads the Dump chutes to the CMC but in CSC mode, it is the responsibility of the CSC/Host to redirect all items.

(5)	The CSC/Host sends a Ready telegram to inform the CMC that the CSC/Host is ready to download data and receive sorting information.

(6)	The CMC requests carrier data from the CSC/Host. As mentioned, this is only done if the CSC/Host is the master of the carrier data and that a hard reset is performed.

(7)	The CSC/Host sends carrier data for all occupied carriers to the CMC.

(8)	When no more carrier data must be sent, a Carrier Data Transmitted telegram is sent from the CSC/Host to the CMC.

(9)	The CMC sends a Ready telegram to inform the CSC/Host that the CMC is ready to start the sorter and sortation.

(10)	The CMC informs the CSC/Host of the status of all chutes.

#### CSC hot standby switchover

When the CMC is connected to a hot standby CSC, all channels are connected to both master and standby CSC.

The messaging on the routing channel looks like this in a switchover situation:

(1)	When the CMC receives the Connected message on the standby channel, it knows this channel is the new master channel.

(2)	The new master CSC transmits "Connected" to CMC. This message may be used by the CMC to stop inducting/discharging items (if not already done when CMC loses connection to the master CSC). Whether it should be done is based on an inspection of the physical layout of the sorter.

The CMC clears CSC alarm points in CMC memory.

(3)	CMC transmits ”Connected” to CSC (on both connections as every telegram).

(4)	When CMC is ready to resume operation it transmits ”Resume” to CSC.

(5)	CSC sends ”Item Redirect” for certain items. Other items just keep their destination.

(6)	CSC sends "Resume" to CMC - CMC enables induct/discharge etc again, if stopped.

(7)	CSC transmits "Set Output" for all outputs controlled by the CSC.

(8)	CSC sends ”Set Status” since all alarms are re-evaluated when switching over.

The Connect and Resume messages act as delimiters in the message flow the same way the ResetComplete and Ready messages do in normal reset flows.

In between Connected and Resume, the CMC continues sending item events to the CSC, but the answer is probably delayed until after the CSC has sent Resume. In case an item has the destination “any scanner”, this will mean that the CMC will scan the item multiple times. Instead, the CMC should park the item on the sorter after a good read.

#### CSC hot standby switchover in a reset sequence

If the CSC switchover happens in the middle of a init seq before both CMC and CSC have transmitted ”Ready”, the new master CSC resets the CMC again. This happens after sending “Connect” to make the CMC know the new master channel. This means that the CMC must be able to handle a Reset req in the middle of the reset telegram scenario.

This is an example of different CSC switchover scenarios while doing a CMC reset:

### Standby state

When the standby is able to take over sortation, it reports “Host Standby State (active)” to CMC. If CMC loses connection to standby, it regards the standby inactive and requests “Host Standby State” when keep alive is up and running again.

### Item inducted from induction unit in keyboard mode

The CSC/Host places a code in the code queue of an induction and the reception of the code is acknowledged by the CMC. When an item arrives in the PEC at the code belt, the CMC will send a Code Used telegram and later an Item Inducted when the item has reached the sorter.

### Disabled induction unit

The induction unit is blocked from the CST or SCADA system through the Command and Control interface (Protocol P1). It is possible to switch between dispatch end keyboard mode while the induction unit is blocked.

If the induction unit is started from the CST/ SCADA system, the CMC will report that start is not possible due to the blocking status of the induction unit.

### Item inducted from induction unit – OH scanner after code PEC at induction unit

In case an overhead scanner is placed at the induction unit, the item will be scanned when passing the scanner. The result will be forwarded to the CSC/Host as soon as the result is received from the OH scanner. This means the CSC/Host may receive the result before an ‘Item Inducted’ telegram has been sent from the CMC.

If the scanner is placed before the induction belt, the carrier number will be 0 (zero) in the Scanner Result. If the scanner is placed after the induction belt, the Scanner Result will contain the number of the carrier on which the item will be placed.

(1)	The item will be inducted with the scanner destination if the CSC/Host has not redirected the item before it is inducted.

(2)	The ‘Item Redirect’ telegram may be sent before the item is inducted (before (1)).

#### OH scanner after code PEC at induction unit – induction unit error

(1) 	If the item does not reach the sorter before an error occurs at the induction unit (in this example an E-stop), the item will be cancelled by sending a ‘CodeNotUsed’ telegram – the CSC/Host must disregard the item.

(2)	If the CSC/Host redirects the item anyway, the item redirect is rejected.

#### OH scanner at induction unit – no redirection from CSC/Host

- If the CSC/Host does not redirect the item before a checkpoint is reached on the sorter, the CMC will inform the CSC/Host and redirect the item to the NoRead dump chute.

### Item inducted from induction unit in scanner/dispatch mode

First the CSC/Host sets the induction unit in dispatch mode to a scanner. The dispatch mode is acknowledged. All items arriving at the induction unit will now be inducted and directed to the scanner. The induction unit will remain in dispatch mode until the CMC receives a new Set Induction Unit Mode telegram with Keyboard mode for the induction unit in question.

### Item inducted, scanned, sorted and supervised

The following scenario shows a normal telegram exchange for an item that is inducted and scanned by a scanner placed above the sorter and then sorted to the destination and supervised by a stray parcel supervision (SPS).

The item is inducted in dispatch mode to scanner.

### Stray item on induction unit

The following scenario shows telegram exchange for a stray item that is inducted from an induction unit.

### Stray item on sorter inducted

The following scenario shows the telegram exchange for a stray item that is inducted from an SPS.

The SPS is in keyboard mode.

### Code queue cleared at induction unit

The following scenario shows how the code queue at an induction unit or MAI is cleared. The CSC/Host is informed about each individual code cleared in a Code Not Used telegram.

### Chute full and max. recirculation event for item

The following scenario shows the recirculation of an item in case the destination chute is full. When the item has recirculated max. recirculation times the CSC/Host is informed. The max. recirculation counter is set to 3.

When the CMC is detecting a Max.Recirculation event, the CSC/Host is informed in an Item Sortation Event telegram. In case the CMC is allowed to redirect items on its own, the new destination will be included in the Item Sortation Event telegram (in this example destination 10).

Whether the CMC is allowed to redirect items on its own is configurable (= Host mode). If the CMC is not allowed to redirect items (= CSC mode), the original destination will be included in the Item Sortation Event telegram.

The redirection of the item at (2) is not necessary. The CMC will dump the item in the Max. recirculation dump chute of no redirection is done.

### No activation event for item

After an item has been discharged, the subsequent SPS will monitor whether the item is still on the carrier or not. If an item is found, an Item Sortation Event telegram with reason 2 (No activation) is sent to the CSC/Host.

If the CMC does not receive a redirection at (1), the CMC will continue to sort the item to its originally destination until max. recirculation. Hereafter the item is dumped in the max. recirculation dump chute.

### Disabled carrier

If a carrier is disabled while carrying an item that has not been discharged, a discharge supervision telegram will be sent from the SPS. The telegram status will be Carrier Disabled. The CMC will mark the carrier as disabled but occupied. When the carrier is enabled, the item will be inducted again via the SPS.

### Upload of index status

The CSC/Host may request upload of index status for all known items in the CMC. When the last index has been transmitted, an ‘Index Status Transmitted’ telegram is sent.

### Upload of dump chutes status

The CSC/Host may request upload of the dump chutes for one or all induction units in the CMC. The following scenario shows the upload of all dump chutes of the CMC.

### Use of ‘Code-Multi Dest’ telegram

The following scenario shows the use of the ‘Code-Multi Dest’ telegram. The CSC/Host may specify a number of destinations and a mode for how the CMC must handle the received destinations. In the following examples ‘mode = nearest’ and up to 6 destinations (3,8,10 and 24) are used.

The CSC/Host sends the telegram with the 6 chute destinations to the CMC (1). In this example the chute destination 0024 is the nearest.

The item is inducted with this destination (2). The chute destination 0024 becomes full (3) and when the item arrives at the chute, an ‘Item Sortation Event’ is sent to the CSC/Host (4) to inform that the item cannot be sorted and the new chute destination ‘0003’ is included. Even though the original chute destination 0024 becomes free (5), the CMC will try to sort the item to the next chute destination 0003.

The item cannot be sorted to chute destination 0003 and 0008 due to chute full (6). The item is finally sorted to chute destination 0010 (7).

### Use of ‘Item Redirect-Multi Dest’ telegram

The ‘Item Redirect-Multi Dest’ telegram may be used to redirect an item already inducted to the sorter and let the CMC decide where to sort the item. The behaviour of the item will be as outlined in section ‎5.2.16.

### Use of ‘Item Transfer' telegram

The Item Transfer telegram is used to hand over item information when an item is transferred from a conveyor line to an induction unit.

The system may look like depicted in the following drawing:

The following telegram scenario is done in connection with the transfer:

Before or when the item reaches the transfer point on the conveyor, the CSC/Host will send a Code telegram with the destination for the item to the induction unit in question.

The conveyor line will inform the CSC/Host about what item is going to be transferred (not part of this protocol scenario).

When the item is transferred (this is detected by the CMC by hardware and handshake digital signals (HS0, HS1)), the CMC informs the CSC/Host of the index of the item and the handshake signals. Now the CSC/Host is responsible to ensure that the correct item is transferred by checking the index and handshake number. If an error is detected, the CSC/Host may redirect the item. This redirection may occur at any point while the item is present at the induction unit and at the sorter.

The CMC/induction unit will induct the item in the normal way.

See also [TRANSFER] for details.

## Telegram Scenarios for Test Operation

Currently, no scenarios are defined.

## Telegram Scenarios for CMC - MES

Please consult the [MES] document for further information.