|<

Control System

CMC Interface Specification Item Transfer

sortercmc.s10

Journal:	s10_Itemtransfer_Specification_DHL eComm DFW & CLT.docx

Date:	04 Dec 2023

Revision:	1.1

Pages:	141

Initials:	DTP

© BEUMER Group 2020 This document and all data contained herein are the exclusive intellectual property rights of BEUMER Group and are supplied in strictest confidence under the express condition that any reproduction, transmission, transcription etc., or storing in any retrieval system/form, or any kind of supply to others is strictly forbidden without express prior written consent. The receiver guarantees that any disclosed material shall not be used in any way detrimental to the interest of BEUMER Group.

Contents

Page

1.	revisions	3

2.	Introduction	4

2.1	About this Document	4

2.2	Target Group	4

2.3	Options	4

2.4	References	5

3.	Terminology	6

4.	​​​​​Item Transfer at Induction units	8

4.1	Item Transfer with Data Exchange	8

4.1.1	Error handling	12

5.	Item Transfer at Chute Destinations	13

5.1	Item transfer with Digital Interface	13

# revisions

| Rev. | Date | Init. | Description |
| --- | --- | --- | --- |
| 1.0 | 25 Sept 2023 | DTP | First Draft |
| 1.1 | 04 Dec 2023 | DTP | Updated tables with project specifics (sorter, inductions, chute and conveyor names and indexes) |

# Introduction

## About this Document

This document describes the process of handing over items in the following situations:

- From a conveyor line to a BEUMER Group induction unit

- From a chute at the BEUMER Group sorter and to a conveyor line, in a controlled manor.

The document describes several transfer processes:

Conveyor line to induction unit:

- Item transfer with digital interface

Chute to conveyor line:

- Item transfer with digital interface

## Target Group

This document is intended for BEUMER Group's sales department and CMC software project managers. It may also be appropriate for external business partners and customers who need a detailed specification of the standard CMC item transfer functionality.

## Options

This document describes the functionality of a standard CMC. Part of the description is marked as options.

Functionalities marked as “option”, are functionalities which are not supported by the standard version of the CMC software. That means that they are either not implemented or must be modified or included in the CMC standard software according to the specific project requirements.

The project-specific functional specification will contain information on which options is/are implemented in a specific CMC project.

## References

​​​​​​​[ROUTING]	Routing Interface

CMC5 Product Release

Journal: p3_Protocol-CMC-Routing DHL eComm DFW & CLT.docx

# Terminology

This section contains a list of definitions and explanations of terms used in the document.

| Carrier | The general designation for the various parts on which items are placed, when transported around the sorter. Carriers are mounted on carts. Examples of carriers are trays and belts. |
| --- | --- |
| Cart | The part of the sorter holding the carriers. A cart can have several carriers. |
| Chute | The exit point of the items on the sorter. |
| CMC | Crisplant Machine Controller |
| CR | Conveyor Ready. CR is a digital signal from a conveyor line attached to a chute. The CR signal indicates that the conveyor is ready and able to receive an item from the chute. When the CR is set low the chute may not transfer any items. It is typically used in combination with RNI and NIP digital signals. |
| CSC | Crisplant System Controller. The logistical control (e.g. code conversion). It includes the same tools as the CST |
| e.g. | exempli gratia (latin) – for example. |
| FAT | Factory Acceptance Test – test performed at BEUMER Group before commissioning on site. See also SAT. |
| Host | A system not supplied by BEUMER Group that can either control the logistics of the sorting process or is used for information exchange by the BEUMER Group system. |
| i.e. | id est (latin) – short for ‘that is’. |
| IU | Induction unit. The entry point of the items to the sorter. |
| IR | Induction unit Ready. IR is a digital signal from an induction unit. The IR signal indicates that the induction unit is ready and able to receive an item from an attached conveyor line. When the IR is set low the conveyor line must stop transferring items immediately. It is typically used in combination with RNI and NIP digital signals. |
| Items | The physical units handled by the sorter. |
| MES | Manual Encoding System (previously known as Crisplant Encoding Unit (CEU)). A PC-based system that enables the operator to register items manually. |
| NIP | Next Item Proceeding. NIP is a digital signal from a conveyor line attached to an induction unit or from a chute with a conveyor line attached. The NIP signal indicates that a new item is proceeding towards the induction unit or the conveyor line at a chute. It is used in combination with an RNI signal to control the item flow between different units. |
| PEC | Photoelectric Cell. The PECs are used to detect items on different locations along the sorter. |
| RNI | Ready Next Item. RNI is a digital signal either from an induction unit or a conveyor line attached to a chute, indicating that the induction unit or conveyor line is ready to receive an item. It is used in combination with an NIP signal to control the item flow between different units. |
| SAT | System Acceptance Test – final test on site performed in co-operation with the customer to ensure that the system fulfils all requirements specified. |
| Stray item | An object detected on one of the sorter trays or induction units, which according to the control system software picture of the trays and induction units should not be occupied. |

# ​​​​​Item Transfer at Induction units

## Item Transfer with Data Exchange

It is possible to transfer item data from the conveyor system to the CMC by using this interface.

Figure 4.1-1 Item transfer system overview

The interface described in this section concerns the transfer of items at position B.

The interface uses the same signals as described in section Error! Reference source not found. and three digital handshake signals – HandshakeNo0, HandshakeNo1 and HandshakeNo2 between the conveyor system and the induction unit. Furthermore, the data to be transferred are also included in the interface; these data are called 'Project data'.

Figure 4.1-2 Schematic overview of the conveyor – induction unit interface with Data

Besides the interface signals described in section Error! Reference source not found. the interface also contains the following for the data exchange.

|  | Conveyor System - IU | Meaning |
| --- | --- | --- |
| 1 |  | HandshakeNo0 |
| 2 |  | HandshakeNo1 |
| 3 |  | HandshakeNo2 |
| 4 |  | Project data |

- The handshake signals form a three-bit handshake number for an item being transferred. The handshake number is used for ensuring correspondence between the items being transferred between the conveyor system and the induction unit in the CSC/host. The handshake signals should be used sequentially, e.g. (1,2,3,…7,1,2,3,...7...). HandshakeNo0 is the least significant bit (e.g. HandshakeNo0 = 0, HandshakeNo1 = 1 and HandshakeNo2 = 0 => Handshake Number = 2). Handshake Number = 0 is an invalid handshake number and indicates an error at the interface.

- The project data are the actual data which needs to be transferred from the conveyor system to the induction unit. This data can be changed from project to project, based on what is needed, but will in most cases consist of a Global Identification Number also called GID number.

The interface involves a third party, e.g. the CSC or an external host as shown in the following picture:

Figure 4.1-3 Transfer data system overview

The transfer of items from the conveyor feed system to the induction unit follows the description and the drawing below. Telegrams referred to in the description are explained in detail in the document [ROUTING].

- The IR signal must be high to transfer an item. When the induction unit is ready to receive the next item, it sets the RNI signal high. The conveyor system PLC is only allowed to start a transfer of an item as long as the RNI signal is high.

- When the conveyor system PLC is ready to start transfer of an item, it sets HandshakeNo0, HandshakeNo1 and HandshakeNo2 to their proper values and updates the 'Project data'. Then it sets the NIP signal high and starts the transfer. The conveyor system must inform the CSC/host about the item and the handshake number (item at position (1)).

- When the induction unit detects that the NIP signal is going high, it sets the RNI signal low and starts tracking the item until the first photocell on the induction unit. The induction unit reads the handshake number when it sees that the NIP signal is high. At this time the item is still on the conveyor system, so the conveyor system shall keep on running to finish the transfer of this item.

At the same time the induction unit informs the CSC/host, by means of an ‘Item Transfer’ telegram, containing the handshake number and an index for the new item. The CSC/host compares the data from the conveyor system and the induction unit and checks that the handshake number is correct and that it is the correct item that is being transferred. The index information must be used by the CSC/host to assign a sorter destination for the item. The CSC/host is allowed to assign the destination for the item from the moment the CSC/host has received an Item Transfer telegram.

- When the conveyor system PLC detects that the RNI signal is low, it shall set the NIP signal low. The induction unit checks that the NIP signal goes low before the item reaches the first photocell on the induction unit. If the NIP signal does not go low before the bag reaches the first photocell on the induction unit, the induction unit will stop and generate an error message.

- The conveyor system must keep the HandshakeNo0 / HandshakeNo1 / HandshakeNo2 signals and the 'Project data' in their valid states until the RNI signal has gone low. When the RNI signal goes low, the handshake signals and the 'Project data' must be set to 0.

- The CMC will receive the item at the code PEC at the induction unit (see Figure 4.1-2 – item at position 2) and start inducting the item to the sorter – the CSC/host is informed by a ‘Code Used’ telegram.

- The CMC is now ready to receive a new item from the conveyor line.

Figure 4.1-4 Handshake from conveyor system to induction unit.

The following overview shows the telegram exchange:

The ‘Item Redirection’ telegram may be sent at any point after the Item transfer telegram but before the item is sorted to its destination.

### Error handling

The conveyor system sets the handshake signals and sends the same value in the telegram to the CSC/host. The CMC looks at the handshake inputs and sends the value of these inputs in the telegram to the CSC/host.

If these two handshake values from the conveyor system and the CMC are not the same, the CSC/host has to handle the item as an unknown item because the interface failed. If it happens for many items there could be a problem with the interface signals or the interface relays.

In case an item is detected at the code PEC without the NIP signal has been detected, the CMC will send an ‘Item Transfer’ telegram with the handshake number set to 8 – legal values are normally 1..7. The CMC will handle the item as a stray item.

The induction unit will monitor the time that the item spends travelling from the NIP signal is set and until the item is detected at the code PEC on the induction unit. The item must arrive in a predefined time window in order to be handled. If the item arrives outside the window, the item will be identified as a stray item. The original item will be cancelled by sending a ‘Code Not Used’ telegram to the CSC/host.

    - Interface Mapping

| Induction Name | Induction Reference-CMC | Sorter | Conveyor PLC. Conveyor Line. Element |
| --- | --- | --- | --- |
| IU01 | 0 | CBS001 | =CS01.FL105.CL111+CV10 |
| IU02 | 1 | CBS001 | =CS01.FL106.CL121+CV10 |
| IU03 | 2 | CBS001 | =CS01.FL107.CL131+CV10 |
| IU04 | 3 | CBS001 | =CS01.FL108.CL141+CV10 |
| IU05 | 4 | CBS001 | =CS01.FL201.CL211+CV10 |
| IU06 | 5 | CBS001 | =CS01.FL202.CL221+CV10 |
| IU07 | 6 | CBS001 | =CS01.FL203.CL231+CV10 |
| IU08 | 7 | CBS001 | =CS01.FL204.CL241+CV10 |

# Item Transfer at Chute Destinations

## Item transfer with Digital Interface

If items must be tracked from the chute to a conveyor line, an RNI/NIP interface may be used.

The chute has three digital signals towards the conveyor system.

|  | Chute – Conveyor System | Meaning |
| --- | --- | --- |
| 1 2 3 |  | Conveyor Ready (CR) Ready Next Item (RNI) Next Item Proceeding (NIP) |
| 4 |  | Full PEC Signal (FPS) |
| 5 |  | Sorter running in Automatic (SRA) |

- The 'Conveyor Ready' signal that indicates that the conveyor is started and able to receive items from the conveyor system, i.e. no errors prevent the conveyor from running. The name of the ‘Conveyor Ready’ signal is abbreviated CR.

- There is a ‘Ready Next Item’ output signal from the conveyor line towards the chute. The signal indicates whether the conveyor line is ready to receive the next item or not. The name of the ‘Ready Next Item’ signal is abbreviated RNI.

The CMC is only allowed to start a transfer of an item into the chute as long as the RNI and CR signals are high. The RNI signal goes low during transfer of an item, but the chute shall continue to transfer the item.

- There is a ‘Next Item Proceeding’ input signal from the chute towards the conveyor line. The signal indicates that the chute has started a transfer of a new item. The name of the ‘Next Item Proceeding’ signal is abbreviated NIP.

Figure 5.1-1 Schematic drawing of chute - conveyor interface.

The transfer of an item from the chute to the conveyor line is to follow the description and drawing below:

- The Conveyor Ready signal must be high as long as the conveyor is able to receive items.

- When the conveyor line is ready to receive the next item, it sets the RNI signal high. The CMC is only allowed to start a transfer of an item as long as the RNI and CR signals are high.

- When the CMC/Chute is ready to start a transfer of an item, it sets the NIP signal high and starts the transfer.

- When the conveyor line detects that the NIP signal is going high, it sets the RNI signal low.  At this time the item is still in the chute.

- When the CMC/chute detects that the RNI signal is low, it shall set the NIP signal low.

- There is a ‘Full PEC Signal' output signal from the chute towards the conveyor line. The signal indicates the unfiltered signal of the full PEC of the chute which is mechanically mounted on the conveyor but electrically connected to the chute. The name of the ‘Full PEC Signal' signal is abbreviated FPS.

The conveyor control uses this signal as wakeup- in case (NIP) was not received, and as jam detection at the 90-degree transfer from the chute slide to the transport conveyor.

- There is a ‘Sorter running in Automatic ' output signal from the chute towards the conveyor line. The signal indicates the sorter is running in automatic and it is used by the conveyor system to start/stop the line connected to this chute. If the signal is TRUE, then the conveyor will start the line. Once the signal goes to FALSE, meaning that the sorter is stopped or not running in automatic mode, then the line will stop. The name of the ‘Sorter running in Automatic ' signal is abbreviated SRA.

    - Interface Mapping

| Chute Name | Chute Reference-CMC | Sorter | Conveyor PLC. Conveyor Line. Element |
| --- | --- | --- | --- |
| CHU001 | 0 | CBS001 | =CS01.NRL01.CL101+CV01 |